BPQI.DOC	July 12, 1993

Written by:

David Farrell N1HAB

34 Vista Drive

Danvers MA 01923

(508) 887-3795 (Topsfield MA)

Compuserve:73530,2701

BPQI is a intercept TSR which inserts itself between the BPQ code and the

application calling the BPQ code.  All calls to the BPQ code are monitored

and a flag is set as to which functions are being used.  Additionally all

streams can be monitored to indicate which streams were used.

Note that if you just run the switch without and application (ie BBS),

then BPQI won't display anything.

To run BPQI, just type bqpi after you install BPQCODE. If you try to install

it again, and "Already Installed" message will appear.  To remove BPQI you

run the included remove.exe program.

BPQI Command line parameters:

BPQI -r#	Displays monitor parameters starting on row #.

BPQI -s#	Monitor only stream #, default=255 indicates all streams!

(If trace file capture is enabled the following commands are avaiable)

BPQI -c		Start file capture on connect to node.

BPQI HotKeys:

LEFT CTRL + LEFT SHIFT R	Reset Capture flags. (low beep)

LEFT CTRL + LEFT SHIFT C	Compression Toggle.

mid then high beep is on.

low beep is off.

What BPQI displays:

The first (and if one stream, only) line shows a memory of all

BPQ functions called.  If a character is highlighted, then that function

number was called.  Left to right are functions 0..15.  Note that the 10;s

digit is not displayed so the middle zero represents function 10, then 1

following this zero is function 11, and so on.

At any time hit the reset hotkey to reset the highlighted numbers.

If you monitor all streams then the second line displays any BPQ references

to that stream.  Highlighted shows the stream was referenced.  You may

reset this line.

The third line shows the connected status.  If a stream is connected it

shows C, if a Session control function was performed, displays an S,

and if disconnected shows a D. You may reset this line too.

To remove the BPQI monitor program, you must exit the terminal program

(ie PAC4 or SYSOPH) and at DOS type REMOVE.  When the TSR is removed a

short beep will be heard.

BUGS?

BPQI is a simple utility to help me test various applications for BPQ

interface.  As such is may have bugs which I haven't discovered so the

simple reason all bugs get through, its impossible to test all conditions.

I would appreciate notification regarding bugs or errors, or possible

additions.  I have a version which records a trace file or all functions

and arguments to disk and will send it to those interested.  I don't include

that here because I haven't found a good way to limit the amount of data

saved, BPQ gets called very frequently!

Source code is also available, to compile to EXE you need to buy the TSR

library "CodeRunner" from Omega Point (a fantastic product from a company

with the best customer support I have ever come across, yes even better

that Hewlett Packard!).  I only use the basic CodeRunner package.

Some Additional notes on TSR's and IRQ's:(Assuming DOS5 or DOS6)

When at DOS you can always type MEM to display available ram.  Do this

before running BPQCODE, then before running BPQI. Note the available memory

at each stage.  When BPQI is removed, more DOS memory is available.  You

may also run Microsoft Diagnostics (MSD.EXE), or Quarterdecks Manifest

(MFT.EXE) to examine interrupts and who is using them.  Under MSD's 'Q'

command BPQI will show using the timer & keyboard, while BPQCODE will show

using the com ports (if node HDLC card).  MFT's first meg, then interrupts

command is more informative showing BPQI using vectors 8,9,28 and 7f.

BPQCODE uses only int 0c or d for COM1 or COM2.  If you remove BPQI then MFT

will show BPQCODE also uses 8 and 7f.  MSD on the other hand show BPQCODE

owns 0 (timer tick) when BPQI is removed.  NOTE that MSD shows HARDWARE IRQ

lines while MFT shows the actual 80x86 vector.  To convert one to the other,

you add 8 to hardware IRQ numbers.  Thus the timer hardware interrupt is 0

but it uses the (8+0) = 8th 80x86 interrupt vector.  MSD also has a TSR

command.

If you load BPQCODE or BPQI (or both) high, MSD is incapable of figuring out

who owns what vectors or memory.

MFT is much better in this regard.  Both the "first Meg" r (programs) and

i (interrupt) commands will show you where (lo or high) in memory your

programs are loaded.  (I am assuming that if you have MFT you are running

QEMM 6 or 7 (yes QEMM v7.0 is out and it looks good!) ).

By the way, both MSD and MFT (especially) MFT are extremely handy utilitys.

Get to know one or the other well.  You can determine everything from TSR

memory info to COM and LPT port status.