TFLINK.DOC

----------

This file contains minimal startup instructions for TFLINK.EXE.  It

will be written properly when I get the time, and if necessary.

TFLINK.EXE is a TSR program that connects a currently running TFPCX

TSR to the second serial port on a DOS system.  A Linux (or other) PC

can then plug into this second port, and use the DOS PC and Baycom

modem combination together as a "virtual" hostmode TNC.

This is the normal way that TFPCX is used with DOS

+-------------------+

| DOS:              | COM1

| SP/GP... -> TFPCX | <------> Baycom <--> Rig

+-------------------+          modem

And here is the way it can be used with Linux, via TFLINK

+-------------+              +-----------------+

| Linux:      | ttyS1   COM2 | DOS:            | COM1

| TNT         | <----------> | TFLINK -> TFPCX | <------> Baycom <--> Rig

+-------------+              +-----------------+          modem

To set this up, first load TFPCX on DOS, with the command

TFPCX -PCOM1 ...other options...

This starts TFPCX running with the Baycom modem connected to port

COM1.

Next, load TFLINK on DOS with

TFLINK -PCOM2

This will start the transfer of data between COM2 and TFPCX running on

the default interrupt of INT FD.  Serial line speed is 9600 baud by

default (can be set by options to TFLINK), 8 bits, no parity, 1 stop bit.

Use 'TFLINK -H' for a full listing of TFLINK options.

It should now be possible to connect to TFPCX as a "virtual" TNC from

Linux.  A simple test terminal shell script, tfterm, is provided -

this shell script can be run on Linux to test the connections.  Once

this combination is working, it should then be possible to run Tnt on

Linux.

When the session is finished, TFLINK can be unloaded.  To unload the

TSRs, it's important to remove them in reverse order.  To remove

TFLINK, use

TFLINK -U -PCOM2

The -P option is mandatory with all usages of TFLINK except -H.

Then TFPCX can be unloaded with

TFPCX -U

Notes:

- Speeds over 9600 baud can cause problems on my system, for reasons

as yet unknown.  9600 baud is fine for Tnt use in any case.

- TFPCX 2.1 is the version I use.  I tried with an earlier one, and

Tnt was not able to sync with it; maybe some disagreement between

TFPCX 1.10 and Tnt?

- You can load such things as the TFPCX 'MYCALL' either from a

TFPCX.INI file on the DOS PC when starting TFPCX, or from the TNC

initialisation built in to Tnt (or whatever Linux terminal you are

using).

Have fun.  Best regards,

73 de Simon, G0FRD

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

G0FRD               | AX25 BBS Packet  - G0FRD @ GB7BEN.#43.GBR.EU

Simon Baldwin       | AMPR.NET TCP/IP  - g0frd@g0frd.ampr.org

Leighton Buzzard    | INTERNET at work - simonb@sco.COM

Beds (IO91PV, SP92) | INTERNET at home - Probably never

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~