NATTA V1.1                                      G7OCW@GB7MAX.#28.GBR.EU

=======================================================================

Purpose:

Chat server for BPQ Nodes. Add this talker to your winpack system!

Example a user connect to your node types CHAT and can have a conversation

with anybody else that is on the chat server...

=======================================================================

Installation:

* Copy all the files into a directory set up chat.cnf

this contains a one line welcome message and your name on the

next line

* This has been compiled for application number 2 so your bpq

applications section (bpqcfg.txt) would look like the following.

APPLICATIONS=BBS,CHAT,what ever else you run.

NOTE Chat needs to be second in the list, it's the way it is when

it's programmed. Also don't forget to run bpqcfg.exe after the

alteration to bpqcfg.txt

If you running winpack BBS with most likley be your winpack system.

* RUN the program using the pif file otherwise windows will not

multitask the program correctly.

* The pif file of course needs to be setup you need to pass two

parameters which are 5 9

9 = number of chat ports

You have 9 conference channels (i.e. /c 1 to 9)

5 = start 9 ports from port 5

Start from your BPQ stream 5 winpack is normally on 1

================== ADDITIONAL HELP ON PARAMETERS ===================

WIN 3.1

1..Load the program called pif editor

2..Load up the pif for the chatserv.pif

3..Then where it says Optional Parameters put 5 9

WIN 95

1..Using Windows Explorer click ONCE on chatserv.exe

2..Then Do File -> Properties

3..Click on the program tab

4..Then where it says cmd line c:\winpack\chatmod.exe

5..Put 5 9 after it so it reads c:\winpack\chatmod.exe 5 9

Obviously your directory could well be different but same pricinpal

=======================================================================

This was Based on G8IMB's Chat Server