TN211_18.DOC

ROUTING LOOPS

*****************

When  TheNet  receives  a command instruction to connect to  a  destination

node,  it  will attempt to set up a circuit via the path it has determined  has

the highest  path quality listing.  If this results in a timed out  connection,

it then  will attempt a connect via the next best route.  If this attempt  also

fails, the node will attempt a third path if one is listed.

Sometimes  a  primary  route  may experience  temporary  failure.   A  node

receiver  down  the line may be subject to interference or desensitization  for

a period.  If this occurs while a connect request is in  progress, its possible

the local node will time the circuit out and attempt a connect on the next best

route listed.  If after a time an unusual delay is noted in having the  connect

request processed, disconnect.  Then, reconnect  to  the local  node and  do an

"N <destination nodecall>"  command.   (Where destination nodecall is the alias

of the destination node one just attempted a connect to).

Notice  the  position  of  the activity arrow on  the  resultant  response.

Chances  are,  the  activity  arrow  will  show an attempt is being made via  a

"routing  loop" path.  A path that has a "0" path quality, or perhaps to a path

that  leads  elsewhere.   As  long  as the activity arrow  is  pointing  to  an

unconnectable  path,  there  is little one can do but wait until  the  internal

timer  "resets" (usually 15 minutes following the last connect request for that

particular  node).  Then if the circuit is clear, one should be able to make  a

connect via the primary path.