GridPoint

from

Northern Lights Software

GridPoint (GP) is a program to calculate bearing and distance between your

location and another grid square or latitude/longitude.  In addition, it

will turn your azimuth rotator if your have the Kansas City Tracker.

You may run GP in several ways:

(1) GP may be called from within RealTrak by pressing <Alt-G> from any

of the text-mode tracking screens.

(2) To run GP as a "normal" program, type GP at the system prompt.

(3) GP may be installed as a pop-up (TSR) program.  Type GP MEM at the

system prompt to install it this way.  Then, press <Alt-G>

from (almost) any text-mode program to pop up the GP window.

NOTE: If you intend to run GP from within RealTrak, it is

better to not have GP installed as a TSR.  Also, some

programs do not tolerate TSRs very well, and GP may not be

stable as a TSR in all programs.

If you don't like <Alt-G> as your hot key to pop up GridPoint,

you can use another key instead.  Make a simple ASCII text file

containing the "scan code" of the key you want to use, and call

file GP_SCAN.DAT.  Scan codes may be easily determined by running

the program SCANCODE.COM provided with the Kansas City Tracker, or

you may look scan codes up in most computer programming manuals.

NOTE: The scan code must be in Hexadecimal, but do not use a leading

$ or trailing H.  For example, if you wanted to use <Alt-Z> as your

GridPoint hot key, your GP_SCAN.DAT file would contain the characters:

2C and nothing more.  The <Alt> key is always assumed to be part of

the hot key.  If GP_SCAN.DAT is missing or invalid, <Alt-G> is

the default.

GridPoint presents you with a small box in the upper right corner of your

screen.  If you are running GP as a TSR, you can move the box with the

cursor arrow keys.  You are prompted for Grid Square.  You may enter either

a 4- or 6-character grid square and GP will calculate the bearing and

distance from your location to that grid square.

You can enter Lat/Lon instead of grid square.  Just type the other

location's latitude (decimal degrees) instead of grid square.  GP notices

that your entry is not a grid square and it will prompt you for longitude

next.  That location's grid square will be calculated and displayed.

If you have the Kansas City Tracker and have installed DRV.COM, you can use

GridPoint to turn your antennas.  Just Press <Ins> to begin turning.  If

you want to stop before the antennas have reached their goal, press <Del>.

You may type in a specific bearing by first pressing <=> (the "equals" key).

Then type the bearing that you want.  As soon as your press <Enter>, the

azimuth rotator will start turning.  This assumes that you have the KCT

installed, of course.

To exit GridPoint, press <Esc> and your previous screen will reappear.  If

you want a reminder of the GP commands, Type GP HELP at the system prompt.

To remove the GridPoint TSR after it has been installed, type GP KILL.

Remember, GridPoint reads your location from the RealTrak file VHFQTH.DAT.

Consequently, GP.EXE must be in the same subdirectory as RealTrak and its

data files.

Have fun with GridPoint!

Northern Lights Software

Michael R. Owen, W9IP

Star Route, Box 60

Canton, NY  13617

(315) 379-0161 (6-9 pm Eastern)

GridPoint

copyright 1992 Michael R. Owen, Ph.D.

Thanks to Tom Manfred (NS2Z) and Russ Wicker (W4WD) for helpful suggestions.

MRO 10/24/92