------------------------- VPTSR for JVFAX ---------------------------------

My hat goes off to DK8JV both for an outstanding job on the JVFAX program

and for the great idea of interfacing the program using Terminate and Stay

Resident device drivers. A TSR is a program that loads itself into memory and

waits to be called by other DOS programs. In this case, JVFAX calls a TSR to

take care of the hardware specific tasks such as getting and receiving pixel

values and operating the PTT relay.

VPTSR is a Terminate and Stay Resident program that allows you to use JVFAX

software with your ViewPort VGA interface. I have had this version on

the air for a few nights and I must say that I have received some of the best

pictures that I have ever seen on SSTV. I hope you will enjoy dusting off your

ViewPort Interface and putting it back on the air using the JVFAX software!

I thank all of you who purchased or built ViewPort interfaces for your patients

with me. My job required that I commute from Indiana to New Jersey almost

every week during 1994. That left little time for working on VPVGA and

delivering on the new features that you all desired. On the bright side, many

late nights in my hotel room were spent learning to program for Windows on my

laptop. Since you are in good hands with DK8JV, I will probably discontinue

development of VPVGA. The last release was 3.85. Instead, I hope to find time

to design a new hardware interface that can be used in the Windows environment.

If you have not already registered your ViewPort VGA software, you can help

defray the cost of the PASCAL compiler required to develop VPTSR and show your

support for this anf future projects by sending $12.00 to:

John Montalbano

10646 E 106th Pl.

Carmel Indiana

46033

PC boards, kits and assembled ViewPort VGA units are available from:

A&A Engineering

2521 W. LaPalma

Unit K.

Anaheim California

92801

Voice (714) 952 - 2114

Fax   (714) 952-3280

********************************** OPERATION ***********************************

Make sure to check the alignment of your ViewPort interface using the vpscope

program provided with VPTSR. I will include a new alignment program

very soon that will help you correctly align the box to receive the VIS signals

at 1100 and 1300 Hz. VIS IS NOT SUPPORTED IN THIS VERSION.  The box will detect

vertical sync, so if you are in the correct mode, the picture will reset to the

top of the screen when the station begins transmission.

If you are using a VESA TSR program such as TLIVESA to achieve 32K colors,

run it first.

Run VPTSR.EXE from the command line before loading JVFAX. The program defaults

to LPT1 at address 0378h. If you use LPT2, run VPTSR as follows:

vptsr -p278

The -p option tells the TSR to use LPT2 at 278. If your printer port is at

address 03BCh, run VPTSR as follows:

vptsr -p3bc

The program will indicate that it has been loaded correctly.

TSRs occasionally will interfere with other programs that you try to load

after loading the TSR. If you have this problem, you can unload VPTSR as

follows:

vptsr -u

You will probably not need to worry about the interrupt (IRQ) used by this

program unless you have another TSR loaded that uses SW interrupt FAh. This

is unlikely.

You will only have to run VPTSR once unless you power down your computer.

After the TSR loads successfully, run JVFAX. The main screen should indicate

that the ViewPort VGA TSR was recognized at FAh. Switch to the configuration

menu and make sure that the Receive and Transmit Parameters are set as follows.

Demodulator   - 8 bits parallel

addr          - 0278,0378 or 03bc depending on how you loaded VPTSR.

IRQ           - [-]

LSB-SSTV-SYNC - YES

Modulator     - 6 bits parallel

addr          - 0278,0378 or 03bc depending on how you loaded VPTSR.

Bdrate        - Don't Care

Dtarate       - Don't care

I suggest that you write a Batch file which does all of the above automatically:

vptsr -u    (To unload vptsr if it is already loaded)

tlivesa     (To load the VESA driver)

jvfax       (To run JVFAX)

-------------------------- File Conversions From TGA ---------------------------

I have included a shareware copy of the Alchemy file conversion program and

a batch file called jvtga.bat. These two programs allow you to read and

transmit Targa files. The program will scale the Targa files up or down to the

340 X 256 size required by JVFAX.

The batch file assumes that alchemy is in a directory under JVFAX called

alchemy. Thus, the batch file looks like this:

.\alchemy\alchemy.exe %1 -t0 -Xa340p -Ya256p -24

If you install alchemy somewhere else, change the batch file to point to

alchemy.exe.

-------------------------- VESA Driver for 32K Colors -------------------------

I have included the program TLIVESA.COM for those of you who have Tseng Labs

32K color cards. Run this program before running JVFAX. Then, set JVFAX for

640 X 480 VESA 32K colors in the configuration menu. No promises here, this

works for my Vanilla Tseng Labs Hicolor Card.

--------------------------------------------------------------------------------

If you have any problems with this program and cannot get help from other HAMS]

on the air, please feel free to contact me after 8:00 Central Time at

(317) 844-2987.

Your ready to go! See you on the radio