G1NNA Multi Port, Multi user

BBS software

V3.0.4

(C) Brian Lloyd B.Sc. (Hon)  G1NNA 1989-1997

SYSOP USER MANUAL

Manual by G.B. Lloyd G1NNB

This software is not public domain. It is only for the use of

licensed amateurs who licensed BBS.  The software must not be

altered or tampered with in any way.You are strongly advised

to register as a user so you can receive notice  of  updates

and additions.  Registration can be sent to G1NNA @ GB7NNA

.DEFINDEX OVERVIEW

OVERVIEW

========

SERVERS, FILTERS and USER PROGRAMS

====================================

NNA software will support servers, filters and user programs. Examples code

is given for those wishing to write their own servers, filters and user

programs. It should be remembered when writing these programs that they

should be small and fast as the multi tasking has to stop to run them.

Servers

----------

NNA supports servers triggered by messages. Servers should be small, fast

programs (since all multi-tasking stops while they are running). They  can

be used to automatically return information to the message originator. All

servers must be listed in a file called SERVERS.BBS stored in the server

directory. The format of that file is:

SERVER_NAME      Description

SERVER_NAME      Description

e.g.  REQDIR    Request directory server

REQFIL     Request file server

When a personal message is received on the BBS, and the BBS is the

destination (i.e. the message isn't going to get forwarded on) the SERVERS

.BBS  file will be checked to see whether there is a server with the same

name as  the To field of the message. If there is, the server will be run.

The output  from the server should be written to a file called SERVER.OUT,

stored in the  server directory. The format of that file should be:

SP to_field @ at_field  Subject  Body of message  /EX

Arguments:

Name of the server directory

From field

@ field

Name of the files directory

Message subject (with any @ field stripped off)

Name of mail directory

Message number that has triggerd this request.

Returns:

0 : Server run successfully (originating message will be killed)

1 : Error

Filters

---------

Filters should be small, fast programs (since all multi-tasking stops while

they are running). They can be interactive, since they can ask for data from

the connected user and be called again when that data has been received. The

various filters are listed in the CONFIG.BBS file and all must reside in the

SERVER directory. Filters can be run at Pre/Post-connection, Pre/Post-message

and Pre/Post file transfer.

Details as follows:

Pre-Connection Filter

-------------------------

Arguments:

Level (0-120)

Output file name

User flags

Callsign of user

User name

Received data (one word per argument)

Returns:

0 : Allow connection.

1 : Disconnect user.

2 : Wait for data from user then call filter again, incrementing level.

3 : Wait for data from user then call filter again, without incrementing level.

4 :  Called again, level incremented without waiting for input from user.

5 :  Called again, without level incremented without waiting for input from user.

Any text output from the filter to the output file will be sent to the user.

If the value 2 is returned from the filter, then the BBS will wait for the

user to send a line of data (up to 60 characters terminated by a carriage

return) then increment the value of level, and call the filter again. If the

value 3 is returned, then the BBS will wait for the user to send a line of

data and call the filter again, without incrementing the level. If a

server returns the value 4 then it will be called again with the level

incremented without waiting for any input from the user. If it returns the

value 5 then it will be called again without the level being incremented and

without waiting for any input from the user. These are useful for servers

that take a long time to run, as they allow the BBS to service other users.

Post-connection filter

----------------------

Arguments:

Level (0-120)

Output file name

User flags

Callsign of user

User name

Received data (one word per argument)

Returns:

0 : Allow connection

1 : Disconnect user

2 : Wait for data from user then call filter again, incrementing level

3 : Wait for data from user then call filter again, without incrementing level

4 :  Called again, level incremented without waiting for input from user.

5 :  Called again, without level incremented without waiting for input from user.

Any text output from the filter to the output file will be sent to the user.

If the value 2 is returned from the filter, then the BBS will wait for the

user to send a line of data (up to 60 characters terminated by a carriage

return) then increment the value of level, and call the filter again. If the

value 3 is returned, then the BBS will wait for the user to send a line of

data and call the filter again, without incrementing the level. If a server

returns the value 4 then it will be called again with the level incremented

without waiting for any input from the user. If it returns the value 5 then

it will be called again without the level being incremented and without

waiting for any input from the user. These are useful for servers that take

a long time to run, as they allow the BBS to service other users.

Pre-message filter

------------------

Arguments:

Level (0-120)

Output file name

User flags

Callsign of user

User name

To field

@ field (including any hierarchical address)

From field

BID

Subject (one argument per word of subject) OR received data (one word per

argument)

Returns:

0 : Allow message creation.

1 : Cancel message creation.

2 : Wait for data from user then call filter again, incrementing level.

3 : Wait for data from user then call filter again, without incrementing level.

4 :  Called again, level incremented without waiting for input from user.

5 :  Called again, without level incremented without waiting for input from user.

Any text output from the filter to the output file will be sent to the user.

If the value 2 is returned from the filter, then the BBS will wait for the

user to send a line of data (up to 60 characters terminated by a carriage

return) then increment the value of level, and call the filter again. If the

value 3 is returned, then the BBS will wait for the user to send a line of

data and call the filter again, without incrementing the level. The first

time the filter is called, the subject is passed, on the second and

subsequent calls the data received from the user is passed instead. Note

that this filter is NOT run when a message is forwarded in.

Post-message creation filter

----------------------------

Arguments:

Level (0-120)

Output file name

User flags

Callsign of user

User name

Name of file containing message (including path)

Received data (one word per argument)

Returns :

0 : Message OK

1 : Hold message

2 : Wait for data from user then call filter again, incrementing level

3 : Wait for data from user then call filter again, without incrementing level

4 :  Called again, level incremented without waiting for input from user.

5 :  Called again, without level incremented without waiting for input from user.

Any text output from the filter (normally giving details of why a message was

held) should be stored in the output file specified in the first argument.

If the value 2 is returned from the filter, then any text in the output file

will be sent to the user and the BBS will then wait for the user to send a

line of data (up to 60 characters terminated by a carriage return) then

increment the value of level, and call the filter again. If the value 3 is

returned, then the BBS will wait for the user to send a line of data and call

the filter again, without incrementing the level.

A post-message creation filter (POSTMSG.EXE) is provided. This filter checks  the

contents of each message for words matching those in the file BADWORDS.BBS

and holds the message if any matches are found. Details of the matches are

stored in the file FILTER.ERR. Message held by the filter are flagged with

an F.

Pre-file transfer filter

------------------------

Arguments:

Level (0-120)

Output file name

User flags

Callsign of user

User name

Name of file to be transferred

Type of file transfer (1 = text, 2 = NPF binary, 3 = NPF image, 4 = YAPPC)

Direction of transfer ("U" for upload, "D" for download)

Received data (one word per argument)

Returns:

0 : Allow file transfer.

1 : Cancel file transfer.

2 : Wait for data from user then call filter again, incrementing level.

3 : Wait for data from user then call filter again, without incrementing level.

4 :  Called again, level incremented without waiting for input from user.

5 :  Called again, without level incremented without waiting for input from user.

Any text output from the filter to the output file will be sent to the user.

If the value 2 is returned from the filter, then the BBS will wait for the

user to send a line of data (up to 60 characters terminated by a carriage

return) then increment the value of level, and call the filter again. If the

value 3 is returned, then the BBS will wait for the user to send a line of

data and call the filter again, without incrementing the level.

A Pre-File filter is supplied which can restrict downloads to regular users

only and also limit download times. The config file for PREFILE.EXE is

XFERTIME.BBS which resides in the BBS route directory.

Post-file transfer filter

-------------------------

Arguments:

Level (0-120)

Output file name

User flags

Callsign of user

User name

Name of file to be transferred

Type of file transfer (1 = text, 2 = NPF binary, 3 = NPF image, 4 = YAPPC)

Direction of transfer ("U" for upload, "D" for download)

Received data (one word per argument)

Returns:

0 : Allow file transfer.

1 : Cancel file transfer.

2 : Wait for data from user then call filter again, incrementing level.

3 : Wait for data from user then call filter again, without incrementing level.

4 :  Called again, level incremented without waiting for input from user.

5 :  Called again, without level incremented without waiting for input from user.

Any text output from the filter to the output file will be sent to the user.

If the value 2 is returned from the filter, then the BBS will wait for the

user to send a line of data (up to 60 characters terminated by a carriage

return) then increment the value of level, and call the filter again. If

the value 3 is returned, then the BBS will wait for the user to send a line

of data and call the filter again, without incrementing the level.

User flags

----------

The user flags passed to each filter are in the form of an unsigned integer

bitmask, with values as follows:

2            Expert mode on

4            Remote SysOp

8            User is a BBS

16           Pause option on

32           Excluded from BBS

64           Default to listing in reverse order

128          SysOp password successfully entered

256          User has restricted access  e.g. user flag of 20 means user is

a remote SysOp and has the pause option turned on.

USER PROGRAMS

================

User-developed programs are now allowed, run by using the OS command. Valid

programs must be listed in a file named OSPROGS.BBS in the servers directory.  The

format of that file is:

PROGRAM_NAME      Description

PROGRAM_NAME      Description

e.g.

PROG1     This is a program

PROG2     Another program

PROG3     Yet another program

The OS command with no arguments will list the OSPROGS.BBS file. OS followed

by the program name, or just the program name, will run the program. Programs

should be small and fast (since all multi-tasking stops while they are running). They

can be interactive, in much the same way as the  filters.

Arguments:

Level (0-120)

Output file name

User flags

Callsign of user

User name

Received data (one word per argument)

Returns:

0 : Successful completion

1 : Error

2 : Wait for data from user then call program again, incrementing level

3 : Wait for data from user then call program again, without incrementing level

4 : Called again, level incremented without waiting for input from user.

5 : Called again, without level incremented without waiting for input from user.

The first time a program is called, any arguments entered at the command line

will be supplied as "received data". On the second and subsequent calls

(assuming that the program has asked to be called again), any data received

from the user will be supplied as "received data". A program may send output

to the user by writing to the output file name  supplied as argument 2.

If the value 2 is returned from the program, then the BBS will wait for the

user to send a line of data (up to 60 characters terminated by a carriage

return) then increment the value of level, and call the program again. If the

value 3 is returned, then the BBS will wait for the user to send a line of

data and call the program again, without incrementing the level. If a server

returns the value 4 then it will be called again with the level incremented

without waiting for any input from the user. If it returns the value 5 then

it will be called again without the level being incremented and without

waiting for any input from the user. These are useful for servers that take a

long time to run, as they allow the BBS to service other users.

Note that a user program may have the same name as an existing BBS command,

in which case the program will be run instead of the BBS command, e.g. if

there is a user program called RL, and a user types RL 5, then the program

RL will be run with 5 as its first argument instead of reading the last 5

messages.