HamlET - A Ham Radio terminal Program

By Paul B. Schou - KE6ET

Copyright (c) 1991-93

HamlET v 3.3 is Shareware. Use it and share it (noncommercially) freely. A

Twenty Dollar ($20.00) contribution will entitle you any HamlET updates or,

heaven forbid, bug fixes developed during the following year. Please indicate

what size floppy you use. Updates will be mailed to you. Please mail checks to

the author:

Paul B. Schou

501 Tayman Drive

Annapolis, MD 21403

Equipment Requirements

HamlET will run on any PC compatible computer using DOS 3.0 or higher.

It will probably run on earlier DOS versions but has not been tested

with earlier DOS's.

HamlET will run with a minimum of 128K of RAM. If you use the

optional shelling or program calling procedures enough memory must

be available for them.

HamlET requires a serial port.

Features:

I wrote HamlET because I wanted some things I couldn't find in other

packet terminal programs. I wanted a small program that would run on

a single floppy and still have room to save data. I wanted a program

that would run when called without having to bother with "Press any key

to continue." or other intermediate steps. I also wanted the ability to

run under DESQview or other multitasking software. Lastly I wanted the

ability to use Comm ports one through four and to select IRQ's for ports

three and four.

HamlET does those things on an uncluttered split screen display. It also

can save or send ASCII files, print received stuff, set the TNC clock

and scroll the receive window. HamlET can also be configured to call your

favorite editor or other programs. HamlET uses the SHIFT-F and CTRL-F

keys to send sets of user definable commands or messages to the TNC. This

feature is especially useful with multimode TNC's. HamlET will select and

run in mono, CGA, EGA VGA or SVGA. HamlET is compatible with WB4APR's

Automatic Position Report System (APRS). It will automatically respond to

APRS position report requests when the APRS mode is active.

HamlET has two distinct personalities. For the Ham new to packet or

computers, it is a very simple terminal program that requires minimal

knowledge. For the experienced packet user it is a fully customizable

program with multiple configuration profiles that will run in multitasking

environments.

HamlET does not mess with your TNC!  Commands will not be sent to the TNC

that you do not specify. There is one exception but you can override it.

HamlET uses the default settings of most TNC's except the few commands

explained below.

For The Impatient:

Switch to the drive/directory where HAMLET.EXE is located and type

hamlet

If you are using 1200 Baud between the TNC and the computer and your TNC

is connected to Comm 1, HamlET will run. If it doesn't read on.

The Configuration File:

HamlET allows the user to select a configuration file. The configuration

file allows the user to select Comm ports and Baud rates and to send

commands to the TNC. This file must be in the same Drive/Directory as

HAMLET.EXE. The default configuration file name is HAMLET.CFG.

If the program doesn't find HAMLET.CFG on start up it will tell you so and

will default to COMM 1 and 1200 Baud. The same defaults will be used if the

program doesn't like the numbers in the HAMLET.CFG file. You'll be told if

the numbers are wrong.

If you want to use a config file other than HAMLET.CFG simply type

hamlet urfle.cfg

when you start the program. urfile.cfg can be any file name.

A configuration file has lines of information: A typical file looks

like this:

1           [Line 1 - Comm Port]

1200        [Line 2 - Baud Rate]

4           [Line 3 - IRQ]

\ted.com

+ Xflow Off

+ MY YOURCALL

= [Time]

- CT The terminal is off.

+ CT The terminal is on. Use CTRL-G to ring my bell.

* 3856.00N 07659.79W    Put your Lat/lon here for APRS.

[LAST LINE]

The wording of the comments inside the brackets [ ] is optional as are

the brackets BUT there must be something there. Anything will work as

long as its separated from the number by a few spaces. This applies to

the first three lines and the Equal (=) sign line. The last line must

have something in it that isn't a + - * or = sign.

The first line designates the Comm Port. 1, 2, 3, or 4 are acceptable. If

you use ports 3 or 4 be sure to read the discussion in the Comm Port

paragraphs.

The second line selects the Baud rate between the computer and the TNC.

HamlET will operate with Baud rates of 300, 1200, 2400, 4800, 9600 or

19200. Most users will find that 9600 Baud works fine. Some TNC mailboxes

may not support hardware hand-shaking. If you get garbled data when reading

long messages (3K+) from your mailbox, reduce the Baud rate to 4800 or less.

The third line selects the IRQ or Interrupt Request. This should be set

to 4 if Comm Port 1 is used and 3 if Comm Port 2 is used. See the discussion

in the Comm Port paragraphs for more detail on the IRQ's for Comm Ports

Three and Four.

The fourth line is used to identify an Editor or other program that can be

called while in HamlET. Put the program name in this line. Be sure to

include the .COM or .EXE extension. If the program is not in the Path

include the full pathname. If you plan to use an Editor to prepare packet

messages make sure you use an ASCII editor. DESQview users should read the

DESQview paragraphs.

NOTE: IF YOU DON'T USE THIS FEATURE LEAVE THE FOURTH LINE BLANK.

The fifth line is used to identify a Logging or other program that can be

called while in HamlET. Put the program name in this line. Be sure to

include the .COM or .EXE extension. If the program is not in the Path

include the full pathname. DESQview users should read the DESQview

paragraphs.

NOTE: IF YOU DON'T USE THIS FEATURE LEAVE THE FIFTH LINE BLANK.

The rest of the file lines are sent to the TNC when the program is

started or exited.

If you want your TNC clock set to the computer clock on start up, put this

line in the .CFG file;

= Time

The APRS mode is activated by including a line, preceded by an * containing

your Latitude, longitude and comments. Including this information will start

HamlET in the APRS mode. The Lat/lon format must be followed exactly. The

numbers are in degrees followed by minutes and decimal minutes and the

hemisphere. Include leading zeros to pad the degrees and whole minutes of

longitude to 5 places and Latitude to 4 places. Be sure to use upper case

for the  N, S, E, and W indicators. The sample line:

* 3856.00N 07629.79W KE6ET's Home QTH

Indicates 38 degree 56.0 minutes North Latitude and 76 degrees 29.79 minutes

West longitude. The phrase "KE6ET's Home QTH" will be added to the APRS

formatted Lat/lon info in the position report and BText information. See the

APRS section for more info.

On start up any TNC command may be sent by preceding the command with a

plus sign (+). Thus the line;

+ CT The terminal is on.

sets CText to "The terminal is on."

When you exit the program any line preceded with a minus sign (-) will

be sent to the TNC. Thus;

- CT The terminal is off.

sets CText to "The terminal is off."

Any number of + and - lines may be included in the HAMLET.CFG file as long

as they are after the fifth line. The +, - and * lines may be intermingled.

A sample HAMLET.CFG file is included in HAMLET.ZIP. Edit the file with any

ASCII editor or word processor to reflect your configuration. If you use a

non ASCII word processor to create or modify the .CFG file be sure to save

it as an ASCII file.

Setting the TNC parameters:

HamlET is designed to work with the default parameters of most TNC's.

These settings are changes from typical default settings. Xflow OFF is

mandatory. The next three are highly recommended. The others are

suggestions.

Parameter       Setting

Xflow          OFF      HamlET uses hardware flow control. Xflow

must be off to avoid conflicts. The serial

cable must be a standard cable. A three wire

cable might not work in overflow situations.

Echo            OFF     Turning Echo OFF prevents what you send

from appearing in the receive window.

Flow            OFF     Turning Flow OFF allows incoming data to

be received while you are typing. HamlET

will separate the send and receive data.

LFIgnore        ON      HamlET works with this OFF or ON unless

sending an ASCII file. Turn it ON to be sure.

CMSg            ON      If you want CT messages sent when someone

connects, this must be ON.

NEwmode         ON      Personal preference. I would rather let the TNC

switch back to Command mode after a disconnect.

PIdcheck        OFF     If you get a lot of ASCII stuff from Nodes

turn this OFF to eliminate the trash.

STREAMCall      ON      If you allow multiple users, turn this ON to

help sort them out.

USers           2 - 10  Allow as many users as you want, up to your

TNC's limit.

Using HamlET

HamlET features a split screen display. The top 18 lines are used for the

Receive window The lower part of the screen has five lines dedicated as

the Send window. The windows are separated by a double line with the

computer clock time running on the right side. If the APRS mode is active

a # will appear to the right of the clock. The bottom line or Prompt line

shows what some function keys do.

Any incoming data will appear in the Receive window. The TAB is

set to eight spaces.

When the clock is running, HamlET is ready for use.

Data that has scrolled off the receive window can be reviewed by using PgUp

to enter the scroll mode. The PgUp and PgDn keys then allow scrolling

through the last 10 screens. A flashing prompt will appear on the prompt

line while scrolling is active. Pressing End returns from the Scroll mode

to the normal Receive mode.

Anything typed in the five line Send window is sent immediately to the TNC.

Limited editing capabilities are available by using the Backspace key.

F1-Help

F1 Brings up a help menu that shows information about the special

keys and shows some configuration data. You probably won't need this

too often.

F2-Print

If a printer is connected to printer port 1, F2 will start the

printer. If the printer is not on or if the program detects a printer

error you will hear a beep. If the printer is OK the word "Print" in

the prompt line will flash. Anything received from the TNC will now

print and appear in the Receive window. Pressing F2 a second time will

turn the print function off and "Print" will stop flashing.

F3-Save

F3 is used when you want to save incoming information to an ASCII file.

Pressing F3 will open a small window that asks you to name the file

that information is to be saved to. You may include a full path and

file name. If you just input a file name the file will be saved in the

current directory. If the file exists HamlET APPENDS data to the end of

the file. If the file doesn't exist HamlET will create a new file.

The word "Save" in the Prompt line will flash while data is saved.

Press F3 again to stop the save function.

F4-Send

F4 is used to send prepared ASCII text files to the TNC. The files

can consist of commands for the TNC or text to be sent to another

station. Pressing F4 will open a small window that asks you for a file

name. Enter the file name that you want to send. This feature is useful

for sending prepared messages or for sending brag tapes. For instance

a file named QTH could send your name and address. You may also prepare

messages or a series of messages to be sent to your local bulletin board.

When you send a file using F4, the transmitted text will appear in the

Send window and the flashing words "Any Key" will replace "F4-Send" on

the prompt line. Pressing any key but the Escape key will abort

transmission of the file. The Escape key will also abort the file

transmission but it will also close the HamlET program which probably

is not what you want to do.

If the file length exceeds the TNC transmit buffer size, transmission

from the computer to the TNC will pause. The text in the Send window

will also pause. Transmission to the TNC and the Send window will

continue when the TNC buffers are empty. This is done automatically

by the programs hardware flow control. Transmission from the TNC to the

other station can be monitored by observing the TNC status lights. XFLOW

must be OFF for hardware flow control to operate properly.

F5-Set TNC Time

Pressing F5 will set the TNC clock to the computer time. This feature

is an alternative to using the "= Time" line in the HAMLET.CFG file.

The F5 command is not shown on the prompt line but is in the help menu.

F6-Editor

Pressing F6 will bring up the editor or other program that you named

in line four of the hamlet.cfg file.  You may use this function to

access your favorite editor or use it to bring up a DOS shell. HamlET

users have found that naming Vern Buerg's LIST.COM in line four, to

access an editor, provides a powerful tool that can be used for editing

or file manipulation.

LIST is shareware and Copyright (C) 1983-1991 by;

Vernon D. Buerg

139 White Oak Circle

Petaluma, CA  94952

LIST is available from the author and many land line BBS's including:

(707) 778-8944    VOR 24-hour bulletin board

(707) 778-8841    MB  24-hour bulletin board

(415) 994-2944    VOR South 24-hour BBS

TED.COM a Tiny EDitor developed by Tom Kihlken of PC magazine is included

in the HAMLET.ZIP file. This is a very efficient editor that uses less

than 4K of disk space. The documentation for it is also included.

F7-Log

Pressing F7 will bring up the Logging or other program that you named

in line five of the hamlet.cfg file.  You may use this function to

access your favorite logger or any other program that you might want to

use.

F8-APRS

Pressing F8 will toggle the Automatic Position Reporting System (APRS)

compatibility mode on or off. When the APRS mode is ON, a happy face #

will appear to the right of the clock. See the APRS section for detailed

information.

F10-Temporary exit to DOS

Pressing F10 will cause a temporary exit to DOS. HamlET will remain in

memory as a TSR. Any DOS program can then be run. To return to HamlET,

type exit. Using F10 in a multitasking mode is not recommended unless

adequate memory has been allocated for HamlET plus any DOS programs

that you call with F10.

PgUp-Scroll

Pressing the PgUp key switches the Receive screen to the Scroll mode.

Subsequent PgUp or PgDn key presses allow scrolling through the last ten

pages. Pressing the End key returns to the receive mode. In Scroll mode

the Send window remains active.

SHIFT-F1 through SHIFT-F10 and CTRL-F1 through CTRL-F10

These keys allow access to prepared files.  You may prepare up to 20

files named SF1.TXT through SF10.TXT and CF1.TXT through CF10.TXT. The

content of these files will be sent when the corresponding F key is

pressed while holding the SHIFT or CTRL key. This feature is especially

useful when using a multimode TNC. A series of commands can be sent to

configure the TNC for the desired mode.

Comm Ports

The PC and it's clones were originally designed to handle only two comm

ports. When the need for more serial capability was recognized most PC's

or I/O cards added Comm 3 and Comm 4 at the addresses and using the

Interrupt Requests (IRQ's) as shown. Most software that uses Comm 3 and 4

also expects these addresses and IRQ's.

Port		Base Address	IRQ

Comm1	   03F8	         4

Comm2	   02F8		 3

Comm3	   03E8		 4

Comm4	   02E8		 3

Unfortunately this causes an immediate conflict between ports 1 and 3 and

between ports 2 and 4. If a modem is connected to port 1, a mouse to port 2

and a TNC to port 3 the modem and the TNC will conflict since they share

the same IRQ. The associated software cannot tell wich port generated the

IRQ. At best the program in use will cause problems and at worst the

computer will hang up.

There is a way to solve the problem. HamlET allows you to select other

IRQ's for comm 3 and 4. If your serial I/O board also allows selection of

other IRQ's (many of them do) all you have to do is pick a number and go.

Be careful picking the number. Stick to 2, 5 or 7. IRQ 2 is usually

for a second parallel port. If you are using two printer ports don't use

IRQ 2. IRQ's 5 and 7 are normally safe if other add-on boards have not

usurped the IRQ's. HamlET accepts only IRQ's 2, 3, 4, 5 and 7. Any other

IRQ numbers will cause an error when the HAMLET.CFG file is read. To make

HamlET respond correctly first set the I/O board to the desired IRQ and

Comm Port. Then edit the first and third lines of the HAMLET.CFG file to

reflect the Port and IRQ numbers.

This procedure applies only to Comm Ports 3 and 4. The IRQ's for Ports 1

and 2 are fixed by the program and are not affected by line three of the

HAMLET.CFG file.

APRS Compatibility Mode

The APRS software developed by Bob Bruninga WB4APR, uses unconnected frames

containing position information to plot station locations on a map. The

position information can contain time, latitude, longitude, course, speed

altitude, weather, comments and other data. HamlET can generate BText lines

and special transmissions containing time, latitude, longitude and comments

in the APRS format. HamlET can also respond automatically to requests for

position information from other stations using APRS or HamlET. The APRS VHF

mode coverage depends on the number of stations participating and on the

same frequency. In the Baltimore/Washington area 145.79 mHz is used by most

APRS compatible stations. All stations are encouraged to set their alias

(MYA) to RELAY, and their UNprotect to APRS V RELAY. If wider coverage is

warranted UN can be set to APRS V RELAY,RELAY or even V RELAY,RELAY,RELAY.

If you want your position data to be sent at regular intervals set your

Beacon time accordingly. Times in the order of every 30 minutes or more are

usually adequate.

The APRS mode is activated by including a line, preceded by an * containing

your Latitude, longitude and comments in your configuration file. The line

will start HamlET in the APRS mode. The Lat/lon format must be followed

exactly. The numbers are in degrees followed by minutes and decimal minutes

and the hemisphere. Include leading zeros to pad the degrees and whole

minutes of longitude to 5 places and Latitude to 4 places. Be sure to use

upper case for the  N, S, E, and W indicators. The sample line:

* 3856.00N 07629.79W KE6ET's Home QTH

Shows 38 degree 56.0 minutes North Latitude and 76 degrees 29.79 minutes

West longitude. The phrase "KE6ET's Home QTH" will be added to the APRS

formatted Lat/lon info in the position report and BText information.

The program will start in the APRS mode and a "Happy Face" # will appear

on the splitline to the right of the clock. Your BText will be set to an

APRS format containing the time you started HamlET, latitude, longitude

and any comments as specified in the configuration file. The following

special keys and subroutines will be active while you are in the APRS

mode and not connected to another station.

F8 - Toggles the APRS mode on or off. The # indicates that APRS is active.

CTRL Q - Transmits the special APRS position report (Q)uery. Any station,

with APRS compatible software, receiving the (Q)uery will respond with

their latitude and longitude.

CTRL T - (T)ransmits your computer time, followed by your latitude,

longitude and comments in an APRS format.

CTRL A - Allows you to (A)dd to or change the lat/lon and/or comments

information. A small window will appear with prompts. Press just Enter at

the prompt to retain the present settings. Be sure to enter the data

correctly. There is no error checking incorporated in this function. Limit

the length of the comments to less than 25 characters.

TAB - APRS provides a method of sending one-line messages to stations within

the digipeat range. The format of the messages is the TO call sign padded

to nine spaces followed by a Colon and the message. To send a one line

message type the callsign and press the TAB key. HamlET will do the padding

and insert the Colon. Type the one line message and press ENTER. Stations

running the APRS software generate and expect acknowledgements to one-line

messages. To send an acknowledgment, type the senders call, press TAB, then

type ack and press ENTER.

When you exit HamlET the BText message time will be reset to the exit time.

The lat/lon data will be followed by a "TNC Only" and whatever comments that

are specified. The BText message can be over written by adding a -BT line

in the configuration file with the text of your choice.

DESQview

HamlET will run in DESQview without problems. Set the memory required to

66k. If you use the F6, F7 or F10  functions when running in DESQview

make sure you set the DV memory required high enough to accommodate the

additional programs. The best practice is to open additional programs as

other DESQview applications. Be sure to configure the serial ports in the

DESQview PIF file to reflect your configuration.

Microsoft Windows

Several Beta testers have run HamlET in Windows without problems. I

cannot verify this from personal experience.

Usual Notices.

HAMLET.EXE, HAMLET.CFG and this document (The Software) is copyrighted by

Paul B. Schou. The copyright owner hereby licenses you to use and

distribute the software with these restrictions:

The program shall be supplied in its original, unmodified form, which

includes this documentation;

For-profit use without a license is prohibited;

The program may not be included, or bundled with other goods or services.

Exceptions may be granted upon written request only.

No fee is charged; An exception is granted to non-profit user's groups,

which are permitted to charge a small fee (not to exceed $5.00) for

materials, handling, postage, and general overhead. No other organization

is permitted to charge any amount for distribution of copies of the

software or  documentation, or to include copies of the software or

documentation with sales of their products.

There is no warranty of any kind. The copyright owner may not be held

liable for any damages, including any lost profits or other incidental or

consequential damages arising out of or inability to use the software.

By using the software, you agree to this.