This is version 1.30 of KAMterm.  Due to my lack of G-TOR capability

here, the G-TOR code is UN-TESTED.  According to the documentation from

Kantronics, I'm sending the right escape sequences, but I always like to

see it work for myself....  If anyone does find any problems with the

G-TOR code, please do let me know about it.

The following new features are added:

1) In the ``send immediately'' mode (i.e., each character is sent to the

TNC as it is typed), a '@' at the beginning of a line will cause you

to switch to the normal input mode for the remainder of that line,

allowing you to use @amtor, @pactor, etc., commands in the

immediate-input mode.

2) @amtor, @pactor, etc., commands supported in function key

definitions.  Basically, anything that starts with an '@' is

assumed to be a macro of some sort.  If it isn't a function key

macro, as such, it is passed to the keyboard input routine based

on the assumption that it's a shortcut command.

3) New function key macro:  @filemac --- allows multiple function key

macros, lines of text, etc., to be put in a file.

USAGE:  @filemac filename

LIMITATIONS:  cannot have nested @filemac commands, still a limit

of 79 chars / line.

4) Not a feature, as such, but a definite improvement:  now, when you

record a conversation to a file, the stupid ``and thus spoke''

nonsense is replaced with a simple >>callsign<< notation.

5) Added PACtor HF Specials to HFSPECIALS (Alt-S) menu that, for some

reason, do not appear to have made it into versions 1.15--1.22.

6) Added G-TOR support with the following:

*) New HFSPECIALS menu option for G-TOR 300 baud

*) New HF specials function key definition:  GT300BAUD

USAGE:  @hfspecials gt300baud

*) Other PACtor HF specials stuff now for G-TOR, too

NOTE:  Binary-mode support for G-TOR didn't make this release---I

need more documentation than I have right now, so that's on

hold for another release....