������������������������������͸

�  THS - TNC Hostmode Server   �

� PHS - PK-xxx Hostmode Server �

������������������������������;

HB9CVV

January 1994

Version 6.1x - OS/2

THS is an OS/2 Packet Radio terminal program to control a TNC-1 or

TNC-2 with the WA8DED or equivalent Firmware.

PHS is an OS/2 Packet Radio terminal program to control a PK-232 or

PK-900 multi-mode terminal node controller. The firmware of the

PK firmware must be dated 30.DEC.88 or later.

Both programs are 32-bit textmode applications and run either in an

OS/2 full screen or in an OS/2 window under OS/2 2.x only.

Major features of THS/PHS

�������������������������

* PHS: support of packet, amtor, rtty, ascii, morse, signal and

pactor modes (depending on firmware).

* PHS: supports both radio modems of a PK-900 simultanuesly.

* user configurable comm-port, colors and texts.

* split screen operation.

* command and parameter entry in mode sensitive dialog windows.

* extended help functions.

* review of received text (backscrolling).

* snapshooting of the review-buffer to file.

* logging (capturing) to file.

* send text from file.

* binary file transfer using YAPP protocol (packet mode).

* multi-channel operation (packet mode).

* heard list showing the path (packet mode).

* Net/Rom frames are decoded (packet mode).

* wordwrapping is available.

* built-in message editor.

* support for screens up to 80*60.

THS contains a subset of the features of PHS because the TNC is

packet mode only.

For PHS, the document describes the operation for the PK-232. A

PK-900 can be operated like a PK-232, but only the first modem

("Radio 1") is serviced in this case. For the 'true' PK-900

operation, please see the appendix.

******************************************************************

* A note to PK-900 users: It is highly recommended NOT to use    *

* the PK-900 at a communication line speed of 19200 baud, but    *

* use 9600 baud only. The PK-900 seems to have problems at 19200 *

* baud (which can be seen by the use of a line monitor).         *

******************************************************************

NOTE

������������

XHS is a hobby project and covers my needs, so I implemented the

features which I wished to have, and did not bother to fit all tastes

and to match all possibilities.  XHS may well be not adequate for

your requirements, please remember however that you are not forced to

use it, nor did you have to pay for it.

Changes and modifications since version 4.03

��������������������������������������������

6.11  Silly bug removed which prevents older firmware which is not

able to understand the newer "HOST 5" command (but only

"HOST Y/N" to be recognized.

6.1   Added support for the 01-DEC-93 firmware from AEA. Users of

older firmware which is not supporting all commands will see

some error messages for the unsupported commands during

startup when PHS loads the PK-xxx parameters. These

unsupported commands are simply marked "unsupported" by PHS

and consequently are not available.

The Help message file has not been edited for the new commands

yet, I am tired of typing manual pages. Anybody willing to do

this (if you look into the help-files, the format is obvious)

is invited to send it to me for incorporation...

Due to the common source code, THS also got the version

number 6.1, but has not experienced any changes.

6.02  Corrected program aborts caused by weird responses of the

PK-900 firmware to CO commands. Using IBM C/C++ Tools now.

6.00  Added support for the PK-900 (two modems). Ported XHS for

compilation with the IBM CSet 32-bit compiler. Version 6 will

now require OS/2 2.x and will not run under OS/2 1.x.

5.00  Added Pactor mode support for the PK-232 01-MAR-93 firmware.

Added word-wrapping for word-oriented (i.e. non-Packet)

modes.

PRINCIPLES OF OPERATION

�����������������������

THS/PHS have been developed from the equivalent, older DOS programs.

The term "XHS" is also used for "THS or/and PHS".

XHS uses the OS/2 Communications driver for the RS-232 line to the

terminal device and make full use if a buffered hardware port is

available.

XHS runs the firmware software exclusively in HOST mode, which

permits a maxiumum of control and cooperation. XHS is written in C

for the IBM C-Set compiler.

The internal implementation of XHS in form of several concurrent

threads. This permits full parallel service of communications

control, screen, and keyboard - you will never notice delays when you

type (opposed to other similar programs).

The windowing system is implemented using the VITAMIN C library (with

modifications), a commercially available software library.  Vitamin C

permits writing to non-active windows, so scrolling in an underlying

window is possible.

This feature, together with the real-time operation, permits to

continue the receiving and displaying of received packets even when

support windows (e.g.  help windows or parameter dialog windows) are

opened.

The PK-232 device or the TNC device are also called "device" or "TNC"

in this document.

INSTALLATION

������������

It is expected that you are familiar with the operation of the TNC or

the PK-232 and with the documents describing that software.

PHS consists of the following files:

PHS.EXE         PK-232 Hostmode Server.

PHS.CFG		Configuration file.

PHSHELP.MSG	Help file.

PHSHELP.IDX	Index file.

XHS6.DOC        Document file

THS consists of the following files:

THS.EXE         TNC Hostmode Server.

THS.CFG		Configuration file.

THSHELP.MSG	Help file.

THSHELP.IDX	Index file.

XHS6.DOC        Document file

The first four files must reside in the same directory. Before you

can run THS or PHS, you MUST edit the configuration file THS.CFG or

PHS.CFG respectively, to reflect YOUR environment, and possibly you

might have to configure your TNC or PK-232 also.  Please refer to the

description of the configuration file at the end of this manual.

The syntax to call THS/PHS is: "THS [filename]", or "PHS [filename]"

where filename is the name of a configuration file. If filename is

omitted, PHS will use the default filename PHS.CFG, and THS will use

THS.CFG.

The TNC and the PK-232 both have a support battery to buffer the

parameter values.  If you use this battery, some TNC/PK-232 parameter

must be preset to values which allow THS/PHS to communicate with the

TNC/PK-232. The communication line parameters of the TNC/PK-232 must

be set to 8 bits, no parity, and the line speed must match the

selected line speed for THS/PHS.

Alternatively, when no battery is connected, PHS will switch the

PK-232 to the selected line speed of PHS.

Though not neccessarily required for hostmode operation THS/PHS

requires the standard 8-wire connection (lines 2-8 and 20) for the

RS-232 communication line.  You can however override this requirement

by a parameter in the configuration file (and loose the detection if

the device is online).

The following chapters describe the operation of PHS. THS is a

functional subset of PHS because a TNC-1 or TNC-2 operates in packet

mode only, so part of the features of PHS are not available for THS.

PHS OPERATION

�������������

After you invoked PHS, you get the PHS opening screen:

�������������������������������������������������������������������������������

����������� PK-232 Host mode Server V6.00 [OS/2] - HB9CVV �����������͸

�                                                                     �

�  Initializing COM3, 9600bd - DSR: on, CTS: on, CD: on, FIFO: yes    �

�  Initializing PK-232 - please wait........done.                     �

�  PK-232 eprom date is: 01-MAR-93.                                   �

�  Loading TNC parameters: done.                                      �

�  Loading CFG parameters: done.                                      �

�  READY. Use ALT/Z for help.                                         �

�                                                                     �

���������������������������������������������������������������������;

�    � 23-MAR 12:34:56

�������������������������������������������������������������������������������

PHS will read the number-of-lines of your current video mode from the

system and adjust its windows accordingly. For all of the screendumps

shown here, the standard of 25 lines has been used.

The single line with the date and time is the statusline, which is

normally invers - you cannot see this in this printout. You also see

that PHS divides the screen into the upper receive window and the

lower transmit window which are seperated by the statusline. The

statusline is different for each mode.  During initialisation and for

a short time during the PK-232 mode switching the statusline is

blanked in the left part.

You can change the colors of all windows (this includes the

statusline) by editing the associated values in the configuration

file.

When this opening screen disappears, PHS switches the PK-232 into the

initial PK-232 mode, which is also specified in the configuration

file.

If the RESETDEVICEONEXIT Parameter of the Config-File is NO

(recommended) the you will realize that PHS temporarily switches the

PK-232 into the morse mode during the initialisation phase. Also,

when you exit PHS, the PK-232 is left in the morse mode.

If "FIFO: yes" is displayed, then the OS/2 communications driver

reported the presence of a buffered asynchronous communictions chip,

and has enabled the fifo-buffer operation of this chip. Note that

non-IBM drivers may not report this, but still use a buffered chip.

THE GENERAL KEYS

����������������

The ESC key is the general help key to call the help window. If you

press ESC in packet mode, then the packet-mode help screen pops up:

�������������������������������������������������������������������������������

�������������������������������� PACKET MODE ������������������������������͸

�                                     �                                     �

� ALT/P   TNC mode param setup menu   � ALT/I   TNC text-param setup menu   �

� ALT/T   PHS parameter setup menu    � ALT/V   View OS/2 directory         �

� ALT/W   Command string directory    � ALT/F   View function-key setup     �

� ALT/L   Logging (capture) on/off    � ALT/R   or # Review (scrollback)    �

� ALT/S   Send an ASCII file          � ALT/K   Call Editor                 �

� ALT/B   Enter urcall (remote call)  � CTRL/U  Erase TX-window             �

� ALT/X   Exit PHS                    � CTRL/X  Erase RX-window             �

� ALT/M   Switch TNC modes            � ESC     Direct command entry        �

� ����������������������������������� � ����������������������������������� �

� ALT/C   Connect menu                � ALT/H   Show formatted heard list   �

� ALT/D   Disconnect                  � ALT/G   Show raw heard list         �

� PgUp    Binary file upload (YAPP)   � INS     Toggle status line display  �

� PgDn    Binary file download (YAPP) � <->     Select Channel              �

�                                     �                                     �

���������������������������������������������������������������������������;

M1234 � VHF � UA:00 RTY:00 � DISCONNECTED               �    � 23-MAR 12:34:56

�������������������������������������������������������������������������������

The help window called by ALT/Z is mode sensitive. The upper half of

the help window (down to the seperator) is identical for all modes

because the keys are valid in all modes and are called general keys.

The lower half presents the keys which are mode specific, and are

called mode keys.

We will now discuss the actions and the support windows caused by the

general keys presented in the upper part of the help window, followed

by a discussion of the individual modes and the mode-related keys and

windows.

Note that when any support window is open and you want to cancel it,

press any key and the window will disappear. If there is no action

from the keyboard, any of these windows will also disappear after

some time.

You will also notice that the activity in the receive window will

continue when any of the support windows is popped up.

ALT/P is a general key and available for all modes. It pops up a

window and shows the PK-232 parameters which are related to the

currently selected mode.  Consequently, the ALT/P window is discussed

individually for each mode later.

ALT/I pops up the PK-232 text parameter setup window:

�������������������������������������������������������������������������������

���� SELECT ��͸

�              �

� (A) CFROM    �

� (B) DFROM    �

� (C) MFROM    �

� (D) MTO      �

� (E) UNPROTO  �

� (F) BTEXT    �

� (G) CTEXT    �

� (H) AAB      �

�              �

��������������;

M1234 � VHF � UA:00 RTY:00 � DISCONNECTED               �    � 23-MAR 12:34:56

�������������������������������������������������������������������������������

To select the command parameter for inspection or change, you can

either press the marked character (A, B, C etc), or move the scroll

bar up and down and then press ENTER (the bar is just not visible in

this printout).  This window will then disappear and the associated

parameter entry window will pop up.

This window will pop up regardless of the current PK-232 mode.

When you select CFROM, DFROM, MFROM or MTO then the associated

parameter entry window pops up - for example:

�������������������������������������������������������������������������������

���� CFROM ���͸

�              �

�     ALL      �

�              �

�              �

�              �

�              �

�              �

�              �

�              �

�              �

�              �

�              �

� <F1>  HELP   �

� <ESC> ABORT  �

� <F10> ACCEPT �

�              �

�              �

M1234 � VHF � UA��������������;CONNECTED               �    � 23-MAR 12:34:56

Valid entries are: ALL / NONE / YES / NO

�������������������������������������������������������������������������������

You can enter a qualifier (ALL, NONE, YES or NO) and a list of

callsigns.

Operation of Support Windows

����������������������������

For all support windows which show the parameter values of commands,

PHS always READS the current parameter values first so you can

inspect the actual values and then apply changes. The cursor is the

positioned on the first input field, and the size of each input field

is visible on the screen (which is not visible in the printouts

here). You move to another commands input field with the down-arrow

and up-arrow keys. For each command you will always get a short help

line on the bottom of the screen. If you need more help about the

command and the parameters, press F1.

With kind permission from AEA, the help texts for the PK-232 commands

have been derived from manual files supplied by AEA and is

copyrighted material.

Finally you may press either F10 to accept the data and apply the

parameter values, or ESC to discard it, in case you changed your

mind.

You must follow the rules for the parameters of a command as

described in the PK-232 manual.  In case of the CFROM window - if you

enter ALL, followed by a callsign list this makes no sense, and you

will get an error message from the PK-232:

�������������������������������������������������������������������������������

���� CFROM ���͸

�              �

�     ALL      �

�              �

�������������������� TNC: �������������������͸

� CFROM - msg from device: "Too many params"  �

���������������������������������������������;

�   HB9DDD     �

�   HB9EEE     �

�   HB9FFF     �

�              �

�              �

�              �

� <F1>  HELP   �

� <ESC> ABORT  �

� <F10> ACCEPT �

�              �

�              �

M1234 � VHF � UA��������������;CONNECTED               �    � 23-MAR 12:34:56

�������������������������������������������������������������������������������

MFROM and MTO is preset to "NO", followed by your callsign. The

reason for this is that PHS monitors when connected, and this

excludes your own packets from the monitor screen. You should not

change this if not absolutely neccessary.

Here is an example of the UNPROTO parameter entry window:

�������������������������������������������������������������������������������

��� UNPROTO ��͸

�              �

� To:          �

�   DB0CZ      �

� via          �

�   HB9PD-7    �

�   DB0HP      �

�              �

�              �

�              �

�              �

�              �

�              �

�              �

� <F1>  HELP   �

� <ESC> ABORT  �

� <F10> ACCEPT �

�              �

M1234 � VHF � UA��������������;CONNECTED               �    � 23-MAR 12:34:56

Callsign and SSID of a digipeater

�������������������������������������������������������������������������������

In this example the cursor is positioned somewhere in the "via" list

of callsigns.

If you selected the BTEXT window, you will get this screen:

�������������������������������������������������������������������������������

������������������������������� BEACON TEXT ������������������������������͸

�                                                                          �

� *** Peter, Qth: Port/Biel - JN37OC                                       �

�                                                                          �

�     <F1> HELP    <ESC> ABORT    <CR> or <F10> ACCEPT                     �

�                                                                          �

��������������������������������������������������������������������������;

M1234 � VHF � UA:00 RTY:00 � DISCONNECTED               �    � 23-MAR 12:34:56

Enter beacon text. Use % to clear it

�������������������������������������������������������������������������������

The initial beacon text has been sent from the beacon string in the

configuration file during the initialisation phase of PHS. It is read from the

TNC for presentation, so you will always see the actual beacon text. Note

however that PHS limits the length of the beacon string to 72 characters.

You will get a similar window for the CONNECT message (CTEXT), and for AAB. The

input field for CTEXT is also 72 characters, for AAB it is 16 characters.

The ALT/V key pops up this window which allows you to specify a OS/2 directory

for viewing, e.g.:

�������������������������������������������������������������������������������

���������������������� Directory ���������������������͸

�                                                      �

� C:\OS2\*.*                                           �

�                                                      �

������������������������������������������������������;

���������������������������������� .... ���������������������������������������

Next you will get something like this:

�������������������������������������������������������������������������������

������������������ DIRECTORY ����������������͸

�                                             �

�  DOS     .SYS      1536   19-DEC-91  08:13  �

�  DOSCALLS.LIB     51200   28-NOV-90  21:19  �

�  E       .EXE     69120   19-DEC-91  06:59  �

�  EAUTIL  .EXE     38400   19-DEC-91  08:19  �

�  EPM     .INI       512   08-JAN-92  20:08  �

�  EXTDSKDD.SYS      2048   19-DEC-91  08:18  �

�  FDISK   .COM    109568   19-DEC-91  05:41  �

�  FDISKPM .EXE     72704   19-DEC-91  05:46  �

�  FIND    .EXE     31744   19-DEC-91  08:22  �

�  FORMAT  .COM     65024   19-DEC-91  08:44  �

�  HELP    .CMD      1024   06-AUG-91  10:05  �

�  HELPMSG .EXE     37888   19-DEC-91  08:27  �

�  HPFS    .IFS    128512   21-DEC-91  02:14  �

�  INI     .RC      18944   08-JAN-92  19:41  �

�  INISYS  .RC       1536   08-JAN-92  19:41  �

�  KBD01   .SYS     28160   19-DEC-91  08:50  �

�          ...more...                         �

M1234 �N�                                             ��     � 24-FEB 16:28:59

���������������������������������������������;

�������������������������������������������������������������������������������

ALT/W pops up the command string directory:

�������������������������������������������������������������������������������

����������� COMMAND STRINGS ���������͸

�                                     �

� (A) C DK1SL VIA HB9F                �

� (B) C HB9SDD                        �

� (C) C HB9PD                         �

� (D) C HB9BRC                        �

� (E) C 4U1ITU-8 VIA HB9PD-7, HB9X... �

�                                     �

�������������������������������������;

M1234 � VHF � UA:00 RTY:00 � DISCONNECTED               �    � 23-MAR 12:34:56

�������������������������������������������������������������������������������

You will see the command string lines from the configuration file

(these are the lines with the COMMANDSTRING keyword).  The number of

command strings is limited to 20, and the length of each line is

limited to 72. Because the width of this window is far less than 72,

all lines which are too long to fit are truncated for presentation,

which is indicated by the "...".

Again, to select a command for execution, you can either press the

marked character (A, B, C etc), or move the scroll bar up and down

and then press ENTER (the bar is just not visible in this printout).

Quite obviously, a line must contain a valid PK-232 command or you

will get an error message when you try to execute it. PHS not allow

the execution of all PK-232 commands - it will refuse to send

commands which would interfere with its operation. Please refer to

the discussion of ESC key.

ALT/F lets you inspect your function key setup:

�������������������������������������������������������������������������������

��������� FUNCTION KEY SETUP ��������͸

�                                     �

� F01  Name & Qth                     �

� F02  Rig SHORT                      �

� F04  Rig LONG                       �

� F05  FEC CQ                         �

� F06  RYRYRY                         �

�                                     �

�������������������������������������;

M1234 � VHF � UA:00 RTY:00 � DISCONNECTED               �    � 23-MAR 12:34:56

�������������������������������������������������������������������������������

You will get the function key title string, not the function key text

which can be much longer than the title, and even contain several

lines.

The function key title strings as well as the function key text

strings are defined in the configuration file.

This window also pops up should you press a function key which has

not been loaded with a string. Whenever you press a 'loaded' function

key, then the associated function key text is inserted in your output

to the PK-232.

ALT/T in any mode pops up the PHS parameter setup window:

�������������������������������������������������������������������������������

�� PHS Parameters �͸

�                   �

� POLLDELAY      50 �

� BELL            1 �

� CBELL           1 �

� WORDWRAP       78 �

� MWINDOW         0 �

� FILTER          1 �

� YPKLEN        254 �

� MAXPFRAM       20 �

� MSTAMP          1 �

�                   �

�  <F1>  HELP       �

�  <ESC> EXIT       �

�  <F10> SET PARAM  �

�������������������;

M1234 � HF  � UA:00 RTY:00 � DISCONNECTED               �    � 23-MAR 12:34:56

Enable the BELL (1/0)

�������������������������������������������������������������������������������

The cursor is positioned at the input field of the first parameter

(BELL).  With any such setup window, you use  the up- and down-arrows

to select  the parameter which you want to change.

As with the ALT/P parameter entry windows, PHS reads the actual

values, lets you apply changes, writes the parameters and then

re-reads them again. If you violated a given range for a parameter,

then no change is done, and this is immediately visible after PHS

reread the parameters.

The PHS parameters are parameters for PHS rather than for the PK-232,

and allow you to:

- change the polldelay of THS/PHS. To avoid an excessive system

load, THS/PHS does not poll the device continuosly but inserts

small delays between successive polls. You may somewhat finetune

Systemload vs THS/PHS performance here.

- enable or disable if you want to hear 'bells' sent to you from a

connected station (you will never hear bells from monitored

stations) (BELL).  This parameter is effective in the packet modes

only.

- enable or disable if you want to hear a bell (warble) when you get

a connect (CBELL). This parameter is effective in the packet modes

only.

- set the packet frame length for binary transfers (YPKLEN). This

parameter is effective in the packet modes only.

- set the column position of input word wrapping (WORDWRAP) - a zero

disables wrapping.

- define the window which gets the monitored frames (MWINDOW).  By

default, this is window zero (because channel zero is the monitor

channel).  You can however route all monitored frames to another

channel window of your own choice. This parameter is effective in

the packet modes only.

- filter out non-printable charcters (MFILTER). If this is enabled,

only BELL, HT, LF, VT, FF, CR and the characters with ASCII values

from 0x20 to 0x7E are displayed as is, the filtered charcter are

shown as a small solid rectangle.  If you use a national character

set, MFILTER should be disabled.

The initial setting of MFILTER is defined in the configuration file's

PHS parameter definition line.

YPKLEN needs an explanation. For binary sends, PHS switches the

PK-232 into  the transparent mode and the packet length  is defined

by YPKLEN. That is,  PHS will read YPKLEN slices from the file, add

the YAPP protocol layer and send it to the PK-232 (the PK-232

parameter PACLEN remain always preset to the maximum value of 256

bytes).

ALT/K invokes the online editor of PHS. You are first asked for a

filename of a file which you want to edit. If the filename does not

exist, PHS asks you if you want to create the file. If you confirm,

you can start editing. In the example the filename "ROLF.MSG" was

given and because such a file did not exist, PHS was told to create

the file. Pressing F1 pops up a series of help windows for the

editor:

�������������������������������������������������������������������������������

���������� File: "ROLF.MSG"  Line:005  Col:14  Free=7838  Insert   ��������ͻ

�Hallo Rolf,                                                                �

�                                                                           �

�i������������������������ HELP - Use F2 to exit ����������������������͸   �

�s�                                                                     �   �

�a�                                                                     �   �

� � ESC            Exit the editor. You get the option to save the      �   �

� �                file to disk. If you don't, calling the editor       �   �

� �                again will place you exactly where you left.         �   �

� � INSERT KEY     Toggles insert and overwrite modes                   �   �

�ͳ F8             Toggles whether carriage returns are symbolically    ���ͼ

�                displayed in the edit window                         �

� F9             Toggles editor display from 128 ASCII characters     �

�                to 256 IBM characters                                �

� UP-ARROW       Moves cursor up a line                               �

� DOWN-ARROW     Moves cursor down a line                             �

� LEFT-ARROW     Moves cursor left a space                            �

� RIGHT-ARROW    Moves cursor right a space                           �

�                                                            more...  �

���������������������������������������������������������������������;

M1234 � VHF � UA:00 RTY:00 � DISCONNECTED               �    � 23-MAR 12:34:56

�������������������������������������������������������������������������������

After you finished editing you will save the file to disk. This file

can then be sent by pressing the ALT/S key in any PK-232 mode. You

cannot send an unsaved file. At any time you can edit the saved file

again.

You will not save the file to disk (but keep the text it the editors

buffer) if you temporarliy want to exit the editor and want to

continue editing later. If the editor is called with a previously

unsaved buffer, it pops up with the cursor positioned excatly where

you left before.

When you edit, just realize that the Amtor and RTTY modes do not

support the full ascii character set.

The editor buffer is limited to 8000 bytes which is sufficient for

message editing.

Printing and Logging

��������������������

There is no 'direct' print support for the OS/2 versions of THS/PHS

because the printers under OS/2 are normally spooled and not directly

accessible. It is recommended to log to a file and print that file

later.

ALT/L toggles the logging to a file. Logging can be enabled

individually for each channel which will go to different files. When

you press ALT/L then a window pops up and asks you for the filename

of the file which will receive the logged data. You cannot specify

anything else but the filename, the extension is always "CAn" where

'n' is the channel number. The logfiles will be created in the

current OS/2 directory.

You are offered a filename which is constructed from the current date

and time but you are free to select another filename. The filename

which is offered has the format "MDD_hhmm.CAn" where 'M' is the

hexadecimal number of the month (i.e.  1 to 9 for january to

september, and A, B and C for october, november and december). 'DD'

is the day of the month, 'hh' is the hour and 'mm' is the minute. A

logfile started October 23, at 12:34 for channel 3 would be given the

filename "A23_1234.CA3". Furthermore, if a subdirectory "LOG" exists

in the current directory, then the file is opened in the LOG

subdirectory, otherwise in the current directory.

�������������������������������������������������������������������������������

M1234 � VHF � UA:00 RTY:00 � DISCONNECTED               �  L � 23-MAR 12:34:56

�������������������������������������������������������������������������������

If logging is on for a channel, this is indicated by the logging-flag

"L" in column 61 of the statusline.

When logging is already switched on for the channel and you press

ALT/L then you are asked if you want to stop logging.

Everything which appears on the receive window of a channel will go

to the logfile if logging is enabled for that channel.

ALT/S permits to send any file containing Ascii text. You are asked

for the filename of the file to be sent. This filename is preset to

the filename you specified for editing (if you did edit before) but

can be overwritten.

�������������������������������������������������������������������������������

KANALZUTEILUNG

*** Bulletin-ID: 158902HB9PD ***

������������������������ ASCII FILE SEND �����������������������͸

de HB�                                                                �

� Filename: PRIGCHAN.TXT                                         �

Als B� Sending line 22 of 37 - ALT/A to abort                         �

"USER�                                                                �

ausre����������������������������������������������������������������;

abgerufen.

Interessant dabei duerfte neben dem Call der USER auch sein auf

welcher Frequenz der jeweilige Benutzer die Box erreicht hat.

Die Kanalzuteilung ist wie folgt organisiert:

Total 18 Kanaele

****************

Kanal 1-4      Direktzugang auf 70cm Freqq. 430.675

Kanal 5-9

Store und Forward mit HB9AJ, HB9XC und DB0CZ

M1234 � VHF � UA:05 RTY:00 � INFORMATION TRANSFER       �    � 23-MAR 12:34:56

�������������������������������������������������������������������������������

Note that the line number which is sent refers to the line which is

currently sent to the PK-232, not the line which is actually

transmitted by the PK-232.

In the packet modes, the text scrolls behind the ASCII FILE SEND

window when it is sent. In the other modes, the text scrolls in the

transmit window, and in the receive window the echo will appear from

the PK-232 when the text is transmitted.

ESC pops up the direct command input window:

�������������������������������������������������������������������������������

������������������������������ TNC COMMAND ENTRY �����������������������������͸

� CMD> MYSELCAL               ���� RESPONSE ���͸                              �

�����������������������������ͳ MYSELCAL: HCVV  ������������������������������;

�����������������;

AMTOR LISTEN  � RECEIVE  �  Phasing      �              �    � 23-MAR 12:34:56

�������������������������������������������������������������������������������

You use the 'normal' command mnemonics of the dialog mode of the

PK-232. PHS translates this into hostmode commands, and also

translates the host-mode response back to the dialoge mode mnemonics.

For THS, you must use the mnemonics of the normal TAPR firmware -

this is translated into the (more cryptic) WA8DED mnemonics by THS

and vs.

In the CMD entry window you can inspect ALL parameters and modify

most of them.  Writing of some parameters is disabled however because

a direct execution would interfere with the operation of PHS. In

either case you will get a message like one of the following:

�������������������������������������������������������������������������������

������������������������������ TNC COMMAND ENTRY �����������������������������͸

� CMD> PA   ��������������������� INFORMATION �������������������͸            �

�����������ͳ PACKET: disabled - use ALT/M to switch device modes ������������;

�����������������������������������������������������;

�������������������������������������������������������������������������������

������������������������������ TNC COMMAND ENTRY �����������������������������͸

� CMD> MF������������������������ INFORMATION ����������������������͸         �

��������ͳ MFILTER: disabled - this parameter must remain unchanged  ���������;

�����������������������������������������������������������;

�������������������������������������������������������������������������������

ALT/M in any mode pops up the device mode select screen:

�������������������������������������������������������������������������������

��� DEVICE MODE ��͸

�                  �

�   Packet (V)HF   �

�   Packet (H)F    �

�  Amtor (L)isten  �

�  Amtor (S)tandby �

�   (R)TTY Baudot  �

�     (A)SCII      �

�     (M)orse      �

�     S(I)gnal     �

�  Pa(C)tor Listen �

� (P)actor Standby �

�                  �

������������������;

M1234 � HF  � UA:00 RTY:00 � DISCONNECTED               �    � 23-MAR 12:34:56

�������������������������������������������������������������������������������

To select the PK-232 mode, you can either press the marked character,

or move the scroll bar up and down and then press ENTER. The bar is

not visible in this printout.

The two different modes for packet just use different values for some

PK-232 parameters.

Packet VHF and HF use the parameter values recommended by AEA.

The different packet modes are indicated in the statusline. You will

see "VHF" in place of "HF" if the PK-232 is in packet-VHF.

The FAX and NAVTEX modes are not supported.

The remainder of the general keys will be discussed after the

discussion of the individual modes.

PACKET MODES

������������

There is one difference between the packet-modes and the other modes:

in packet you are offered one monitor channel window and four

connectable channel windows (for all other modes there is just one

receive window). The channels numbers are from zero to four, but

channel zero is marked with an "M" to remind you that this is the

monitor channel. You use the left-arrow and right-arrow keys to

switch the channels. When you switch to a new channel, the full

(upper) receive window is completely replaced by the new channel

window. The status line changes also, reflecting the status of

currently selected window.

The channel indicator is on the leftmost side of the status line. The

current channel is indicated by a reverse letter, that is, one of the

letters of the M1234 string - which is normally (like the rest of the

statusline) dark on cyan - is cyan on black. Also, if you selected

one channel - this is called the active channel and the active

(receive) window - and ANOTHER channel gets input, then the indicator

of that channel changes to white on cyan. Once you selected that

window for inspection, the indicator returns to its normal color.

The monitored channel gets all monitored packets and you cannot

connect on this channel. The statusline for the monitored channel is:

�������������������������������������������������������������������������������

M1234 � VHF �          ��� MONITOR CHANNEL ���          �    � 23-MAR 12:34:56

�������������������������������������������������������������������������������

However you can redirect the monitored input to one of the

connectable channels if you want to see the monitoring there. Please

refer to the MWINDOW command in the ALT/T window.

For the channels 1 to 4 the status line is like this:

�������������������������������������������������������������������������������

M1234 � VHF � UA:03 RTY:00 � INFORMATION TRANSFER       �    � 23-MAR 12:34:56

�������������������������������������������������������������������������������

The number after the "UA:" is the number of unacknowledged frames,

RTY displays the number of retries so far. The next field is the link

state.

Pressing the INS key toggles the status line to the other display

format:

�������������������������������������������������������������������������������

M1234 � VHF � HB9PD via HB9PD-7                         �    � 23-MAR 12:34:56

�������������������������������������������������������������������������������

The UA/RTY and linkstate field is replaced by the connectee. If you

use several digipeaters and the string is gets too long, the

connectee string is truncated on the right hand side.

--------------------------- NOTE ON THS ---------------------------------------

The statusline for THS is like:

�������������������������������������������������������������������������������

M1234 � NS:07 UA:03 RTY:00 � INFORMATION TRANSFER       �    � 23-MAR 12:34:56

�������������������������������������������������������������������������������

Where NS indicates the number of frames which have been already moved

to the TNC but have not yet been sent.

ALT/P in packet mode pops up the packet mode PK-232 parameter setup

window:

�������������������������������������������������������������������������������

��������������������� PK-232 Parameters �������������������͸

�                                                           �

� ACKPRIOR        Y   HID             N   RESPTIME        0 �

� AUDELAY         0   LITE            N   RETRY          10 �

� AX25L2V2        Y   MAXFRAME        4   SLOTTIME       30 �

� AXDELAY         0   MCON            6   SQUELCH         N �

� AXHANG          0   MID             0   TRIES           0 �

� BEACON        E 0   MONITOR         6   TXDELAY        40 �

� CHECK          25   MRPT            Y   USERS           4 �

� CMSG            N   MSTAMP          N   WHYNOT          N �

� CODE            2   MYALIAS    BIENNE   XMITOK          Y �

� CONPERM         N   MYCALL     HB9CVV                     �

� DWAIT          16   PASSALL         N   <F1>  HELP        �

� FRACK           8   PERSIST        84   <ESC> EXIT        �

� FRICK           0   PPERSIST        Y   <F10> SET PARAM   �

� FULLDUP         N   RELINK          N                     �

�                                                           �

�����������������������������������������������������������;

M1234 � VHF � UA:00 RTY:00 � DISCONNECTED               �    � 23-MAR 12:34:56

Audio delay in 10mS (range 0-120)

�������������������������������������������������������������������������������

PHS/THS always reads the parameters from the PK-232/TNC, so you

always get the actual PK-232 or TNC-1/2 parameters. You can now move

the cursor to a command and change the parameter.  To apply the

changes, press F10. PHS then writes the parameters to the PK-232 and

re-reads them again. If a parameter which you have changed again gets

the old value, then you have violated a range check and the PK-232

refused to accept your value. The process of reading, writing, and

re-reading is indicated in the window. It also takes a few seconds,

even when the communiation line to the PK-232 is operated at 9600

bauds.

You will see all parameter which are valid for the packet mode, and

which you may change. You may miss some packet mode commands here -

but the parameters of such commands must remain unchanged at preset

values for a proper operation of PHS.

There are some commands like MRPT and PASSALL which should be changed

only if neccessary, because there is still a possible interaction.

For example, quite obviously, if you surpress the indication of

digipeaters in monitored frames then the HEARD-List of PHS will not

contain only incomplete information. Also, if PASSALL is YES, you can

get 'clobbered' callsigns in the heard-list.

For some commands (BEACON, MCON and MONITOR) you will get a secondary

selection window which offers you discrete choices.

A typical screen from the monitored channel is like this:

�������������������������������������������������������������������������������

HB9X-7*>HB9X-9 [I;3,3]:

NON - Bid 21A90BHB9X deja recu

8>

HB9X-6*>4U1ITU-8 (RR;3)

HB9X-6*>4U1ITU-8 [I;3,2]:

SB ALL    < OH1HS  @ EU $787_ON4HU

HB9X-7*>HB9X-9 (RR;4)

HB9X-6*>4U1ITU-8 (RR,F;3)

HB9X-7*>HB9CED [I;3,2]:

MOL:HB9X-7} Invalid command (CONNECT CQ IDENT NODES PARMS ROUTES USERS)

HB9X-7*>HB9DIG>HB9DIG-7 [RR,P;6]

HB9X-6*>4U1ITU-8 (RR,F;3)

HB9DIG-7>HB9DIG*>HB9X-7 [I;1,6]CF:

�NET/ROM frame: orgin node DB0FRG-2 to dst node HB9X-7, ttl: 24

INFO ACK: ckt 0/238, rxseq 2

HB9X-7*>HB9X-9 [I;4,4]:

NON - Bid 21A90BHB9X deja recu

8>

HB9X-7*>HB9CED [RR,P;3]

HB9X-7*>HB9CED (RR,F;3)

HB9X-6*>4U1ITU-8 (RR;4)

M1234 � VHF �          ��� MONITOR CHANNEL ���          �    � 23-MAR 12:34:56

�������������������������������������������������������������������������������

If something is scrolled off-screen, press ALT/R or Up-arrow to

review it again.  Reviewing is possible individually for all channels

on packet (and for the single channel of the other modes). The lower

(transmit) window will be superseded by:

���������������������������������� .... ���������������������������������������

Cursor on line   0 of 200

REWIEW - Use: # # PgUp PgDn, ESC to exit

ALT/L - Snapshot to file

�������������������������������������������������������������������������������

Use the indicated keys for moving, and ESC to leave the reviewing.

The number of lines which you have for reviewing depends on

parameters in the configuration file. The minimum is 50, the maximum

is about 500. These values can be set individually for each channel.

Whenever you start reviewing, PHS makes a copy the current active

receive window to the review buffer, and then covers the upper 85% of

the receive window to present the review buffer. The "Cursor on line"

gives you an indication where you are inside the review buffer.

During review, receiving to the (mostly covered) receive window

continues.

ALT/L lets you snapshot the review buffer to a file in the current

OS/2 directory. The filename is generated from the date and time and

the extension is always "SNP". For the 23 October at 12:34:56 the

generated filename will be "A23_1234.SNP" (see "Logging and Printing"

for a discussion of the generated filenames). Furthermore, if a

subdirectory "LOG" exists in the current directory, then the file is

opened in the LOG subdirectory, otherwise it is opened in the current

directory.

You press ALT/H to get the heard list formatted and sorted last heard

first:

�������������������������������������������������������������������������������

BBS H�������� HEARD LIST (entries: 10) - XXXX - <Enter> to connect �������͸

�  1. 23-MAR 12:34:56  HB9X-7                                         �

Enreg�  2. 23-MAR 12:34:56  HB9CED-15                                      �

Enreg�  3. 23-MAR 12:34:56  HB9DIG-7 via HB9DIG                            �

�  4. 23-MAR 12:34:56  HB9AC-8 via HB9DIG HB9AC                       �

Pas d�  5. 23-MAR 12:34:56  DG7KAR-8 via HB9DIG DB0DQ DB0GE-2              �

HB9CE�  6. 23-MAR 12:34:56  HB9X-6                                         �

DG7KA�  7. 23-MAR 12:34:56  HB9X-9                                         �

HB9AC�  8. 23-MAR 12:34:56  DG7KAR-7                                       �

HB9CE�  9. 23-MAR 12:34:56  HB9X-14                                        �

HB9X-� 10. 23-MAR 12:34:56  DB1EC-15                                       �

HB9X-���������������������������������������������������������������������;

HB9DIG-7>HB9DIG*>HB9X-7 (RR,F;1)

HB9X-7*>HB9CED [I;1,2]:

tion pour l'instant.

(H)elp (C)heck (L)ist (R)ead (S)end (E)rase (D)ir (U)sage (SP)eak (Q)uit

HB9CED de HB9X>

HB9CED-15*>HB9X [I;2,0]:

C HB9N-7

HB9X-7*>HB9CED (RR;2)

M1234 � VHF �          ��� MONITOR CHANNEL ���          �    � 23-MAR 12:34:56

�������������������������������������������������������������������������������

If you select the raw heard list, you get the original frames

unformatted but still sorted by time:

�������������������������������������������������������������������������������

Enreg�������� HEARD LIST (entries: 10) - XXXX - <Enter> to connect �������͸

Enreg�  1. 23-MAR 12:34:56  HB9CED-15*>HB9X (RR;3)                         �

�  2. 23-MAR 12:34:56  HB9X-7*>HB9CED (RR,F;2)                        �

Pas d�  3. 23-MAR 12:34:56  HB9DIG-7>HB9DIG*>HB9X-7 (RR,F;1)               �

HB9CE�  4. 23-MAR 12:34:56  HB9AC-8>HB9AC>HB9DIG*>HB9X-9 [C,P]             �

DG7KA�  5. 23-MAR 12:34:56  DG7KAR-8>DB0GE-2>DB0DQ>HB9DIG*>HB9X-7 (RR,F;4) �

HB9AC�  6. 23-MAR 12:34:56  HB9X-6*>4U1ITU-8 (DM,F)                        �

HB9CE�  7. 23-MAR 12:34:56  HB9X-9*>HB9X-7 (DM,F)                          �

HB9X-�  8. 23-MAR 12:34:56  DG7KAR-7*>4U1ITU-8 (RR;4)                      �

HB9X-�  9. 23-MAR 12:34:56  HB9X-14*>HB9DIG>HB9AC>HB9AC-8 [I;2,1]:         �

HB9DI� 10. 23-MAR 12:34:56  DB1EC-15*>4U1ITU-8 [D,P]                       �

HB9X-���������������������������������������������������������������������;

tion pour l'instant.

���������������������������������� .... ���������������������������������������

The heard list will contain a maximum of 50 different stations. If

more than 50 stations have been heard, the oldest call is superseded.

You use the arrow keys to scroll within the heard list. A scroll bar,

which again is not visible here is moving - you can position the bar

on an entry on the heard list and simply press the ENTER key to try a

connect. You must first select an unconnected channel, pop up the

heard-list by ALT/H, select the entry and press ENTER.

The arrows are visible in place of the "XXXX" which you see here in

the heard list title.  These little arrows cannot be used in this

printout and had to be substituted.

ALT/C pops up the Connect window, this has the same look-and-feel

like the UNPROTO window which we discussed before. To summarize, if

you want to initialize a connection, you have three choices:

- the Connect-window by ALT/C

- by a selection of the heard-list

- by an entry in the command directory (ALT/W), provided you

have preset connect commands there.

ALT/D is used to initialize a disconnect.

If someone is connecting you, PHS will give a sound (warble). For

incoming connects the PK-232 selects the lowest unconnected channel,

which might not necessarily be the channel which you have currently

selected for display.  You will still realize such a connect because

of the warble, and because the channels indicator is highlighted.

Any text which you type will appear in the lower transmit window. If

word-wrapping is on (which is the default, see ALT/T), your text will

wrap. The input text is sent to the PK-232 when you press the ENTER

key. It is then removed from the transmit window and inserted into

the receive window.  Additionally the text is highlighted in the

receive window, so you can easily distinguish between your text and

the received text.

Note that the transmit buffer of PHS is 250 characters. You will get

the warning "At end of TX-buffer" if you reached the end of the

buffer. You will get the warning "At beginning of TX-buffer" when you

erase characters and no more characters are available for deletion.

BINARY FILE TRANSFERS

���������������������

The next and last chapters for the Packet-modes show how binary

transfers are done. PHS implements the YAPP protocol as defined by

Jeff Jacobson, WA7MBL.  Care should be exercised when employing

binary transfers because upon entry of the yapp transfer mode, PHS

will change the PK-232 parameters to obtain maximum throughput. These

parameters are reset to the previous values when you leave the yapp

transfer mode. To keep friends, yapp-transfers should not be done on

qso channels - use a free channel. You also CANNOT change channels

during a binary transfer, you will have to finish or abort the

transfer first.  Consequently you can invoke a binary transfer on ONE

channel only. Again, this is optimized for throughput and is intended

to provide fast transfers.

Yapp uploading (sending of a binary file) is invoked with the PgUp

key, downloading (receiving) with the PgDn key. PHS will ask you for

a filename first, and then enter the yapp transfer mode. Note that

when you are receiving, you will also have to specify the filename of

a file which will get the data.

Note: YAPP transfers have not been extensively tested in the OS/2

versions.

When sending a binary transfer, you get a window like this:

�������������������������������������������������������������������������������

������ Uploading (YAPP) - ALT/A: Cancel - ALT/K: Kill ����͸

23-MAR-89  12:29:32 �                                                          �

�    FILE: PHS.EXE                                         �

�    Size:  198583                                         �

�    Date: 23-MAR-89 20:41                                 �

�                                                          �

�  Header:  PHS.EXE  198583 bytes, 23-MAR-89 20:41         �

�   Bytes:    11938 read,   10414 sent                     �

�    Rate:       95 bytes/sec, time to go:  33:00          �

�                                                          �

�   STATE:  Sending DATA                                   �

�                                                          �

�   TNC buffer full, waiting.                              �

�                                                          �

����������������������������������������������������������;

M1234 � VHF � UA:06 RTY:00 � INFORMATION TRANSFER       �    � 23-MAR 12:34:56

�������������������������������������������������������������������������������

The bytes read indicate the number of bytes read from the file, the

bytes sent is the number of acknowledged bytes. The message "TNC

buffer full, waiting." indicates that the PK-232s buffer are full and

that PHS is waiting for the PK-232 to become ready for another slice

of data.

Unfortunately the PK-232 does not buffer more than MAXF frames, which

reduces the maximum obtainable troughput to below 100 bytes per

second. With other devices like TNC-2 or the PC*Packet Adapter it is

possible to reach up to 125 bytes per second. Yes, the link is 1200

bauds, but AX-25 is a syncronous protocol, so a single data byte is

not framed with start and stop bits as on asynchronous RS-232 links.

The theoretical maximum is 1200/8 bytes per second, and the AX-25

protocol overhead reduces this number then by at least 15%.

If you want to abort a transfer, you can use ALT/A to cancel it,

which uses the cancel mechanism of the yapp protocol. If you got a

bad link however you will have a TNC full with untransmitted frames

and the frame with the cancel request queued behind. Unfortunately

the only way to flush the untransmitted data in the PK-232 is a

disconnect.

This is why ALT/K has been introduced: ALT/K will kill the transfer

by disconnecting the link and then will re-connect. All of this is

accompanied by messages on the screen.

The YAPP-download window is similar to the upload window. Note

however that the number for the troughput (bytes/sec) will always

differ slightly between receiver and sender - there are always some

data packets on the air.

AMTOR MODES

�����������

ALT/Z in the amtor modes pops up the amtor-modes help screen:

�������������������������������������������������������������������������������

�������������������������������� AMTOR MODES ������������������������������͸

�                                     �                                     �

� ALT/P   TNC mode param setup menu   � ALT/I   TNC text-param setup menu   �

� ALT/T   PHS parameter setup menu    � ALT/V   View OS/2 directory         �

� ALT/W   Command string directory    � ALT/F   View function-key setup     �

� ALT/L   Logging (capture) on/off    � ALT/R   or # Review (scrollback)    �

� ALT/S   Send an ASCII file          � ALT/K   Call Editor                 �

� ALT/B   Enter urcall (remote call)  � CTRL/U  Erase TX-window             �

� ALT/X   Exit PHS                    � CTRL/X  Erase RX-window             �

� ALT/M   Switch TNC modes            � ESC     Direct command entry        �

� ����������������������������������� � ����������������������������������� �

� ALT-C   Start ARQ call              � GRAY +  Start FEC transmit          �

� ALT-Y   Start SELFEC call           � GRAY -  FEC: Receive - immediate    �

� INS     LISTEN: Resynchronize       �         ARQ: Break link - immediate �

�         ARQ: Force changeover       � GRAY *  Clear PK-232 tx buffer      �

� DEL     Force LETTERS case          �   #     Toggle LISTEN <> STANDBY    �

� ������������� in text ������������� � ������������� in text ������������� �

� CTRL-D  FEC: Switch to receive      � CTRL-O  Send LETTERS character      �

�         ARQ: Break link             � CTRL-N  Send FIGURES character      �

� CTRL-F  Like CTRL-D, add CW-id      � CTRL-B  Send "urcall de mycall"     �

� CTRL-T  Send date+time              � END     Send "BTU <CTRL-B> +?"      �

A�                                     �                                     �

���������������������������������������������������������������������������;

�������������������������������������������������������������������������������

The single 'A' visible on the left side is from the covered status

line. If you had setup another screen size before calling PHS (e.g.

80*43), then the statusline would not be covered by the window and

would be visble.

The status line in the Amtor modes is like:

�������������������������������������������������������������������������������

AMTOR LISTEN  � RECEIVE  �  Phasing      �              �    � 23-MAR 12:34:56

�������������������������������������������������������������������������������

where the second field will indicate either "RECEIVE" or "TRANSMIT",

and the first field will show "AMTOR LISTEN", "AMTOR STANDBY" "AMTOR

QSO", "FEC" or "SELFEC". The third field indicates the current PK-232

status and closely follows the LED indicators of the PK-232. The

forth field is always empty. The fifth field will contains the "L"

indicator, when logging is on.

Opposed to the packet mode, where your text is sent to the PK-232

when you press the ENTER key, in non-packet modes your text is sent

to the PK-232 on a word-by-word basis, and is buffered in the PK-232.

The text is not cleared in the lower transmit window but instead it

will scroll there. It is echoed in the upper receive window by the

PK-232 whenever it is sent out (echo-as-sent). The wordwise operation

allows you to correct only the current word. It is not possible to

correct text which has already been sent to the PK-232. For an

indication about what has already been sent to the PK-232, the

already-sent text is highlighted. So if you typed in:

Did you have to apply any modificati

and the cursor is positioned after the 'i', then all previous lines,

and this line up to and including the word 'any' is highlighted, but

'modificati' is not, because it has not yet been sent.

You start a FEC transmission with the GRAY PLUS key (on the keypad).

Normally you will not stop the transmission with the GRAY MINUS key

because the PK-232 will then immediately switch to receive,

regardless of still untransmitted text.  If you switch to receive

with untransmitted text, you will get a warning message from PHS

("transmit data remaining"). This data will be sent the next time

when you switch to transmit. However you have the option to flush the

data by clearing the PK-232's transmit buffer with the GRAY-* key.

Anytime you can insert a CTRL/D or CTRL/F into the text which you

input. The PK-232 will switch to receive whenever it detects such a

character (refer to the PK-232 manual) in the transmit data stream.

Just a note: when you insert a CTRL/D followed by a carriage return

(Enter-key), then you will get the transmit-data-remaining warning,

because after switching to receive by the CTRL/D, the carriage return

is still untransmitted.

To start a Amtor QSO, press ALT/C and you will get a small window

which lets you enter the other stations selcal. For a SELFEC

transmission, press ALT/Y.

You may use ALT/B to pop up a window which lets you enter the remote

stations callsign, and once you did, whenever you insert a CTRL/B

into the transmit text, it is replaced by PHS with the string "

<remote-callsign> DE <my-callsign> ".  My-callsign is from line one

in the configuration file (a possible packet-SSID is removed).

For convenience, in ARQ, the END key inserts the Amtor-changeover

signal "+?" into the transmit text.

If you are in Amtor-Listen mode and you have difficulties to monitor

an Amtor-qso use the INS key to force a resynchronisation. If you are

in an ARQ qso, the INS key forces a changeover.

ALT/P in the amtor modes pops up the amtor-modes PK-232 parameter

setup window:

�������������������������������������������������������������������������������

����������� PK-232 Parameters ���������͸

�                                       �

� ADELAY          4   SRXALL          Y �

� ARQTMO         60   TXREV           Y �

� ARQTOL          3   WIDESHFT        N �

� CODE            2   WRU             N �

� MYALTCAL     HCVV                     �

� MYIDENT   KBPCVVV   <F1>  HELP        �

� MYSELCAL     HCVV   <ESC> EXIT        �

� RFEC            Y   <F10> SET PARAM   �

� RXREV           N                     �

�                                       �

���������������������������������������;

AMTOR LISTEN  � RECEIVE  �  Phasing      �              �    � 23-MAR 12:34:56

ARQ transmit delay in 10mS (range 1-9)

�������������������������������������������������������������������������������

PACTOR MODES

������������

ALT/Z in the pactor modes pops up the pactor-modes help screen:

�������������������������������������������������������������������������������

������������������������������� PACTOR MODES ������������������������������͸

�                                     �                                     �

� ALT-P   PK-232 param setup menu     � ALT-I   PK-232 textparam setup menu �

� ALT-T   PHS parameter setup menu    � ALT-V   View OS/2 directory         �

� ALT-W   Command string directory    � ALT-F   View function-key setup     �

� ALT-L   Logging (capture) on/off    � ALT-R   or # Review (scrollback)    �

� ALT-S   Send an ASCII file          � ALT-K   Call Editor                 �

� ALT-B   Enter urcall (remote call)  � CTRL-U  Erase TX-window             �

� ALT-X   Exit PHS                    � CTRL-X  Erase RX-window             �

� ALT-M   Switch PK-232 modes         � ESC     Direct command entry        �

� ����������������������������������� � ����������������������������������� �

� ALT-C   PACTOR connect menu         � ALT-D   CONNECT: Break link immedia �

� GRAY *  Clear PK-232 tx buffer      � GRAY +  Start UNPROTO transmit      �

� GRAY -  UNPROTO: Receive, immediate � INS     LISTEN: Resynchronize       �

�         CONNECT: Break link immedia �         CONNECT: Force changeover   �

� DEL     CONNECT: Break link         �   #     Toggle LISTEN <-> STANDBY   �

� ������������� in text ������������� � ������������� in text ������������� �

� CTRL-D  UNPROTO: Switch to receive  � CTRL-Y  CONNECT: Changeover         �

�         CONNECT: Break link         � CTRL-F  Like CTRL-D, add CW-id      �

� CTRL-B  Send "urcall de mycall"     � CTRL-T  Send date+time              �

� END     Send "BTU <CTRL-B><CTRL-Y>" �                                     �

P�                                     �                                     �

���������������������������������������������������������������������������;

The single 'P' visible on the left side is from the covered status

line. If you had setup another screen size before calling PHS (e.g.

80*43), then the statusline would not be covered by the window and

would be visble.

The status line in the Pactor modes is like:

�������������������������������������������������������������������������������

PACTOR LISTEN � RECEIVE  �  Phasing      �              �    � 23-MAR 12:34:56

�������������������������������������������������������������������������������

where the second field will indicate either "RECEIVE" or "TRANSMIT",

and the first field will show "PACTOR LISTEN", "PACTOR STANDBY",

PACTOR CONNECT" or "PACTOR UNPROTO". The third field indicates the

current PK-232 status and closely follows the LED indicators of the

PK-232. The forth field is always empty. The fifth field will

contains the "L" indicator, when logging is on.

Opposed to the packet mode, where your text is sent to the PK-232

when you press the ENTER key, in non-packet modes your text is sent

to the PK-232 on a word-by-word basis, and is buffered in the PK-232.

The text is not cleared in the lower transmit window but instead it

will scroll there. It is echoed in the upper receive window by the

PK-232 whenever it is sent out (echo-as-sent). The wordwise operation

allows you to correct only the current word. It is not possible to

correct text which has already been sent to the PK-232. For an

indication about what has already been sent to the PK-232, the

already-sent text is highlighted. So if you typed in:

Did you have to apply any modificati

and the cursor is positioned after the 'i', then all previous lines,

and this line up to and including the word 'any' is highlighted, but

'modificati' is not, because it has not yet been sent.

You start an UNPROTO transmission with the GRAY PLUS key (on the

keypad).  Normally you will not stop the transmission with the GRAY

MINUS key because the PK-232 will then immediately switch to receive,

regardless of still untransmitted text.  If you switch to receive

with untransmitted text, you will get a warning message from PHS

("transmit data remaining"). This data will be sent the next time

when you switch to transmit. However you have the option to flush the

data by clearing the PK-232's transmit buffer with the GRAY-* key.

Anytime you can insert a CTRL/D or CTRL/F into the text which you

input. The PK-232 will switch to receive whenever it detects such a

character (refer to the PK-232 manual) in the transmit data stream.

Just a note: when you insert a CTRL/D followed by a carriage return

(Enter-key), then you will get the transmit-data-remaining warning,

because after switching to receive by the CTRL/D, the carriage return

is still untransmitted.

To start a Pactor QSO, press ALT/C and you will get a small window

which lets you enter the other stations callsign.

As soon as you are connected in Pactor, the other (remote) stations

callsign will appear in the statusline. When you are transmitting

during the qso and want to change-over, type CTRL-Y or press the END

key to get the BTU-string which is:

BTU <remote callsign> de <my callsign><CTRL-Y>

If you are in Pactor-Listen mode and you have difficulties to monitor

an Pactor-qso use the INS key to force a resynchronisation. If you are

connected, the INS key forces an immediate (!) changeover.

ALT/P in the pactor modes pops up the pactor-modes PK-232 parameter

setup window:

�������������������������������������������������������������������������������

� PK-232 Parameters �

�                   �

� CODE            2 �

� MYPTCALL   HB9CVV �

� PT200           Y �

� PTHUFF          1 �

�                   �

� <F1>  HELP        �

� <ESC> EXIT        �

� <F10> SET PARAM   �

�                   �

�������������������;

PACTOR LISTEN � RECEIVE  �  Phasing      �              �    � 23-MAR 12:34:56

�������������������������������������������������������������������������������

RTTY MODE

���������

ALT/Z in rtty mode pops up the rtty-mode help screen:

�������������������������������������������������������������������������������

��������������������������������� RTTY MODE �������������������������������͸

.                                     .                                     .

.                                     .                                     .

� ����������������������������������� � ����������������������������������� �

� GRAY +  Transmit                    � GRAY *  Clear TNC transmit buffer   �

� GRAY -  Receive - immediate         � DEL     Force LETTERS case          �

� ������������� in text ������������� � ������������� in text ������������� �

� CTRL/D  Switch to receive           � CTRL/O  Send LETTERS character      �

� CTRL/F  Like CTRL/D, add CW-id      � CTRL/N  Send FIGURES character      �

� CTRL/B  Send " urcall de mycall "   � CTRL/T  Send date+time              �

�                                     �                                     �

���������������������������������������������������������������������������;

RECEIVE  � RTTY          �                              �    � 23-MAR 12:34:56

�������������������������������������������������������������������������������

ALT/P in rtty mode pops up the rtty mode PK-232 parameter setup

window:

�������������������������������������������������������������������������������

�� TNC Parameters �͸

�                   �

� ACRRTTY        72 �

� CCITT           N �

� CODE            0 �

� CRADD           N �

� DIDDLE          Y �

� RBAUD          45 �

� RXREV           N �

� TXREV           N �

� USOS            Y �

� WIDESHFT        N �

� WRU             N �

�                   �

� <F1>  HELP        �

� <ESC> EXIT        �

� <F10> SET PARAM   �

�                   �

�������������������;

���������������������������������� .... ���������������������������������������

ASCII MODE

����������

ALT/Z in the ascii mode pops up the ascii-mode help screen:

�������������������������������������������������������������������������������

�������������������������������� ASCII MODE �������������������������������͸

.                                     .                                     .

.                                     .                                     .

� ����������������������������������� � ����������������������������������� �

� GRAY +  Transmit                    � GRAY *  Clear TNC transmit buffer   �

� GRAY -  Receive - immediate         �                                     �

� ������������� in text ������������� � ������������� in text ������������� �

� CTRL/D  Switch to receive           � CTRL/F  Like CTRL/D, add CW-id      �

� CTRL/B  Send "urcall de mycall"     � CTRL/T  Send date+time              �

�                                     �                                     �

���������������������������������������������������������������������������;

RECEIVE  � ASCII         �                              �    � 23-MAR 12:34:56

�������������������������������������������������������������������������������

ALT/P in ascii mode pops up the ascii mode PK-232 parameter setup

window:

�������������������������������������������������������������������������������

�� TNC Parameters �͸

�                   �

� ABAUD         300 �

� ACRRTTY        72 �

� DIDDLE          Y �

� RXREV           N �

� TXREV           N �

� WIDESHFT        N �

� WRU             N �

�                   �

� <F1>  HELP        �

� <ESC> EXIT        �

� <F10> SET PARAM   �

�                   �

�������������������;

���������������������������������� .... ���������������������������������������

MORSE MODE

����������

ALT/Z in the morse mode pops up the morse-mode help screen:

�������������������������������������������������������������������������������

�������������������������������� MORSE MODE �������������������������������͸

.                                     .                                     .

.                                     .                                     .

� ����������������������������������� � ����������������������������������� �

� GRAY +  Transmit                    � INS     Unlock receive speed (WPM)  �

� GRAY -  Receive - immediate         � DEL     Lock receive speed (WPM)    �

� GRAY *  Clear TNC transmit buffer   �                                     �

� ������������� in text ������������� � ������������� in text ������������� �

� CTRL/D  Switch to receive           � CTRL/B  Send "urcall de mycall"     �

� CTRL/T  Send date+time              �                                     �

�                                     �                                     �

���������������������������������������������������������������������������;

RECEIVE  � MORSE         � 20 WPM �                     �    � 23-MAR 12:34:56

�������������������������������������������������������������������������������

ALT/P in morse mode pops up the morse mode PK-232 parameter setup

window:

�������������������������������������������������������������������������������

�� TNC Parameters �͸

�                   �

� MSPEED         20 �

� MWEIGHT        10 �

�                   �

� <F1>  HELP        �

� <ESC> EXIT        �

� <F10> SET PARAM   �

�                   �

�������������������;

���������������������������������� .... ���������������������������������������

SIGNAL MODE

�����������

ALT/Z in the signal mode pops up the signal-mode help screen:

�������������������������������������������������������������������������������

�������������������������������� SIGNAL MODE ������������������������������͸

.                                     .                                     .

.                                     .                                     .

� ����������������������������������� � ����������������������������������� �

� INS     Ok - accept & switch        �                                     �

� ��������������� NOTE �������������� �                                     �

�    DO NOT SWITCH TO FAX or NAVTEX   �                                     �

�                                     �                                     �

���������������������������������������������������������������������������;

RECEIVE  � SIGNAL                                       �    � 23-MAR 12:34:56

�������������������������������������������������������������������������������

Please do not switch to an unsupported mode (FAX or NAVTEX), it will

not work.

There is no signal mode PK-232 parameter setup window.

Finally, the secret ALT-key:

Whenever you give ALT/A, any incoming message to the currently

selected window is accompanied by a deep summing tone. Another ALT/A

disables the sound. There is no visible indication of ALT/A, because

obviously there is an audible one.  I often use ALT/A when my partner

says "wait a minute", because I have no klicking relais in my TRX

which would attract my attention after that minute. Obviously, ALT/A

should be avoided in non-packet modes...

THE CONFIGURATION FILE

����������������������

PHS reads the configuration file whenever you start PHS.  The

configuration file contains information about hardware and software

parameters. If this information is not accurate, PHS will be unable

to proceed.

You must edit the file PHS.CFG to reflect your hardware

configuration, callsigns and texts. If you are not able to edit a

text-file you cannot operate PHS.

Here is an example of a configuration file:

�������������������������������������������������������������������������������

Devicetype = 4              ; PK-900.

Callsign   = HB9CVV         ; My callsign

;PTCallsign = HB9CVV/A      ; My Pactor callsign (optional)

Selcall    = HCVV           ; My selcall

Comport    = COM4           ; COMport name

Baudrate   = 9600           ; The baudrate

UseRS232Controls        = YES

ChannelMwindowsize      = 200

Channel1Windowsize      = 200

Channel2Windowsize      = 100

Channel3Windowsize      = 100

Channel4Windowsize      =  50

ChannelM2windowsize     = 200	; Channel M2 and A-D for PK-900 only.

ChannelAWindowsize      = 200

ChannelBWindowsize      = 100

ChannelCWindowsize      = 100

ChannelDWindowsize      =  50

StartupPK232mode	= 2

EditorColumnSize        = 70

EditorRowSize		= 0

MFILTERenabled          = NO

UTCoffsethours		= -1

Polldelay		= 50

ResetDeviceOnExit       = YES

Skip_ParamInit          = NO

CommandString           = C DK1SL VIA HB9F

CommandString           = C HB9SDD

CommandString           = C HB9PD

CommandString           = C HB9PD VIA HB9PD-7

StartupDeviceCmd	= MYALIAS BIENNE

StartupDeviceCmd	= TXREV YES

StartupDeviceCmd	= RXREV NO

StartupDeviceCmd	= TXDELAY 30/50

ConnectMsg = Operator is OFFLINE, this message is recorded...

BeaconText = *** Peter, Qth: Port/Biel - JN37OC ***

F01KeyTitle = Name & Qth

F01KeyText = My name is Peter, the QTH is Port, 20 miles west...

F02KeyTitle = Rig SHORT

F02KeyText = -

The rig here is: a 486/33-PC, PK-900, Yaesu FT-990, 100W to a GP. -

I am using my own PK-232/900 Hostmode Server (PHS) program under OS/2.

F03KeyTitle = Rig LONG

F03KeyText = -

The rig here is:-

-----------------------------------------------------------

Programs  : THS (TNC Hostmode Server - OS/2) V6.0-

PHS (PK-232/900 Hostmode Server - OS/2) 6.0.-

Computers : (1) 486/33, 16MB RAM, 660MB disk, OS/2 2.1.-

(2) 486/33, 16MB RAM, 660MB disk, OS/2 2.0.-

TRX       : FT-990, 100W.-

TW-4100E, 45W.-

Antenna   : DX-2000 GP for HF.-

X-300 2m/70cm vertical.-

TNC       : TNC-1 with TNC-2 board and WA8DED firmware.-

PK-232 with Pactor firmware.-

PK-900 with Pactor firmware.-

-----------------------------------------------------------

In use: PHS, 486/33, PK-900, FT-990, GP.

F05KeyTitle = FEC CQ

F05KeyText = -

4 CQ CQ CQ CQ CQ DE HB9CVV HB9CVV HB9CVV (HCVV)-

3 CQ CQ CQ CQ CQ DE HB9CVV HB9CVV HB9CVV (HCVV)-

2 CQ CQ CQ CQ CQ DE HB9CVV HB9CVV HB9CVV (HCVV)-

1 CQ CQ CQ CQ CQ DE HB9CVV HB9CVV HB9CVV (HCVV)-

SELCAL HCVV HCVV HCVV - PSE KKKKK-

(

F06KeyTitle = RYRYRY

F06KeyText = RYRYRYRYRYRYRYRYRYRYRYRYRYRYRYRYRYRYRYRYRYRYRY

F07KeyTitle = Pactor CQ

F07KeyText = -

4 CQ CQ CQ CQ CQ DE HB9CVV HB9CVV HB9CVV-

3 CQ CQ CQ CQ CQ DE HB9CVV HB9CVV HB9CVV-

2 CQ CQ CQ CQ CQ DE HB9CVV HB9CVV HB9CVV-

1 CQ CQ CQ CQ CQ DE HB9CVV HB9CVV HB9CVV-

PSE KKKKK-

(

ReceiveWindow_Text		= Green on Black

TransmitWindow_Text             = Cyan on Black

StatuslineText                  = White on Blue

Review_Text                     = White on Blue

ServiceWindow_Border            = White on Blue

ServiceWindow_Text              = Blue on lightgrey

CommandEntry_Border             = White on Black

CommandEntry_Text               = Yellow on Red

CommandEntry_InactiveField      = Yellow on Green

CommandEntry_ActiveField        = Yellow on Green

ParameterDialog_Border          = LightGreen on Black

ParameterDialog_Text            = Black on lightgrey

ParameterDialog_InactiveField   = Blue on Lightgrey

ParameterDialog_ActiveField     = White on Blue

ParameterHelp_Border            = White on Blue

ParameterHelp_Text              = Lightcyan on Blue

FileXfer_Border                 = Black on lightgrey

FileXfer_Text                   = White on Brown

EditorWindow_Border             = White on Blue

EditorWindow_Text               = Yellow on Blue

EditorWindow_ActiveField        = Yellow on Red

HeardList_Border                = LightRed on Brown

HeardList_Text                  = White on Brown

;OnScreenChannelId               = LightBlue on Lightgrey

OnScreenChannelId               = White on Blue

OffScreenChannelId              = Cyan on Blue

OffScreenChannelIdData          = Yellow on Blue

SentTextInReceiveWindow         = default

EchoAsSentText			= default

�������������������������������������������������������������������������������

All lines have the format: KEYWORD = PARAMETER and are not case

sensitive. A semicolon must preceed comments. If a line ends with a

dash ('-'), then the dash is replaced by a newline character and the

following line is appended.  This feature is used for multi-line text

string parameters.

There are mandatory and optional keywords. The CALLSIGN keyword

(obviously) is mandatory, CHANNEL1_WINDOWSIZE is not - if you omit an

optional keyword, you get the defaults.

"DeviceType"

Use '1' if you use a TNC-1 with the WA8DED (or equivalent)

firmware, a '2' for a TNC-2 with the WA8DED firmware, a '3'

for the PK-232 and a '4' for a PK-900. Mandatory.

"Callsign" and "Selcall"

You guessed it. Both are mandatory for PHS, for THS, only

"Callsign" is mandatory.

"PTCallsign"

Callsign for Pactor. Defaults to "Callsign".

"Comport" and "Baudrate"

The Comport name and the baudrate to be used. Mandatory.

"UseRS232Controls"

Use 'NO' if you only have a 3-wire connection. Optional, the

default is YES.

"ChannelMwindowsize", "Channel1windowsize", "Channel2windowsize",

"Channel3windowsize" and "Channel4windowsize"

Number of lines for the monitor channel and the four

operating channels. Optional, the default for each is 200.

"StartupPK232mode"

You can select the initial PK-232 mode here. The mode numbers

are 0 to 9 for Packet-VHF, Packet-HF, Amtor-Listen,

Amtor-Standby, RTTY, Ascii, Morse, Signal, Pactor-listen and

Pactor-Standby mode.  This is the mode which the PK-232 should

enter immediately after startup.  Optional, PHS only. The

default is 2 for Amtor-Listen.

If RESETONDEVICEEXIT is NO, you will instead get the actual

device's mode.

"EditorColumnSize" and "EditorRowSize"

Defines the editor window size, columns and rows. The range

for the column is 64 to 79, and includes the window border.

If a given value exceeds one of the limits, then the

according limit value is taken.  If you omit this parameter,

or specify a zero, then the default value of 76 is taken. The

range for the row depends on the size of the (upper) receive

window, which itself depends on the number of lines of your

current video mode. The range if from half that window to

full size minus one. For the standard 25 line mode, the range

is 10 to 20, also including the border.  The default here is

the lowest value, i.e. half the size of the receive window.

"MFILTERenabled"

This determines the initial value of the MFILTER variable,

see the description of the ALT/T command.  "NO" disables the

filtering, "YES" enables it.  Optional, the default (e.g. if

omitted) is NO.

"BELLenabled"

This determines the initial value of the BELL variable,

see the description of the ALT/T command. "NO" disables the

filtering, "YES" enables it. Optional, the default (e.g. if

omitted) is "NO".

"UTCoffsethours"

During startup, PHS gets the current system time of your PC,

adds the "UTCoffsethours", and loads this date and time into

the PK-232. The intention is to have the PK-232 loaded with

the UTC rather than with the local time - in case you want to

use the CTRL/T feature of the PK-232 to send date and time.

Note however that if there is a non-zero UTC offset

specified, then the link status messages (e.g.  "CONNECTED

to") are always time-stamped by the PK-232 and will

consequently show the PK-232s time. This time differs by the

offset hours from the time on the status line of PHS.

The date/time of the PK-232 is refreshed every full hour by

PHS because the PC's clock is more accurate that the PK-232

clock.

Optional, PHS only. The default is zero.

"Polldelay"

To avoid an excessive system load, THS/PHS does not poll

the device continuosly but inserts small delays between

successive polls. You may somewhat finetune Systemload vs

THS/PHS performance here.  The range is 1 to 999 mS.

Optional, the default is 50 (mS). You can change this

parameter also online in the ALT-T window.

Note however that when THS/PHS is run in an AVIO window,

considerable CPU time may be needed for the text display

(depending how fast your video system is).

"ResetDeviceOnExit"

Normally, THS and PHS will reset the device during startup,

to get clean conditions for the price of loosing all stored

information. If ResetDeviceOnExit is NO, then THS/PHS does

not reset the device but tries to derive the devices current

state. This is however not fail safe under all conditions.

The preferred value for this optional parameter is the

default of YES.

"Skip_Paraminit"

If NO, then PHS/THS does not load the parameters during

startup. Not doing this, the device can be in a state

where the operation of PHS will fail. You should set

this parameter only to NO when you have used PHS/THS

for a while and do not use other terminal programs

which might modify these parameters to their own use.

"CommandString"

The parameter is a text string and must contain a valid TNC

or PK-232 command.  Up to 20 "CommandString" lines with a

text not exceeding 72 characters are possible.  They are

stored by THS/PHS and presented together with a selection

character when you press ALT/W.  When you press the selection

character or move the bar to the desired entry, then the

associated string is sent as a command to the on-screen

channel. Intended to build a call-sign directory for

connects, you may however use other commands there as well,

so it is called "Command String Directory". CommandStrings

are optional.

THS users note: The TNC commands use the TAPR syntax, NOT the

more cryptic WA8DED syntax.

"StartupDeviceCmd"

You can specify up to 10 StartupDeviceCmd lines. The

parameter text must not exceed 40 characters and is sent as

command to the TNC/PK-232 when THS/PHS loads the TNC/PK-232

parameters during startup.

This is done *after* THS/PHS has sent the default set of

parameter commands and so allows the user to override the

default parameter set of THS/PHS.  You should however take

some care - the text is sent must be a valid command and can

only contain a command which does not interfere with THS/PHS

operation. The non-interfering commands which appear in the

ALT/P windows. Optional.

"ConnectMsg"

The parameter text is your connect message, and may not

exceed 72 characters, if it does, then the text is truncated.

This text is loaded connect message any time online.

Mandatory.

"BeaconText"

The parameter text is your beacon text, and may not exceed 72

characters, if it does, then the text is truncated. This text

is loaded into the TNC/PK-232 when THS/PHS is started. You

can however alter your cbeacon text any time online.

Mandatory.

"F01KeyTitle" & "F01KeyText" to "F10KeyTitle" & "F10KeyText"

The function key title string and the function key text is

assigned to the appropriate function key. The keytitle string

is a descriptor text (up to 30 characters), the keytext

string cannot exceed 1000 characters. The keytext strings are

intended for reasonable sized texts like station descriptions

or cq calls.

When you press ALT/F then THS/PHS shows the titles of the

text strings in a window box. The titles also come up when

you press a function key which has nothing assigned.

When you press a function key to which a text has been

assinged, THS/PHS will send this text string to the

TNC/PK-232.

The function-key text must contain only printable characters,

and two characters have a special meaning: The '(' character

at the end of a line (and only at the end of a line !) is

translated into CTRL/D when PHS reads the line.  The '<'

character is translated into CTRL/F (if at the end of a

line).  CTRL/D and CTRL/F are used by the PK-232 to switch to

receive when embedded in text.

In the sample CFG-file you find a FEC-cq call. The single

line with the opening-bracket character (which is at the end

of that line) is translated into CTRL/D and the PK-232 will

switch to receive at the end of this cq-call.

All FxxKeyTitle and FxxKeyText keywords are optional.

The COLOR keywords.

The (mandatory) color keywords are used to define the

appearance of THS/PHS. You may alter the parameters to fit

your taste. The parameters have the form

<foreground color> on <background color>

Possible colors are:

Foreground      Background

BLACK		BLACK

BLUE		BLUE

GREEN		GREEN

CYAN		CYAN

RED		RED

MAGENTA		MAGENTA

BROWN		BROWN

LIGHTGRAY	LIGHGRAY

DARKGRAY	|

LIGHTBLUE	|

LIGHTGREEN	|

LIGHTCYAN	|- not for background

LIGHTRED	|

LIGHTMAGENTA	|

YELLOW		|

WHITE		|

"OnScreenChannelId"

"OffScreenChannelId"

"OffScreenChannelIdData"

On the left side of the statusline in the Packet-mode are the

channel indicators: the string "M1234" - for the monitor

channel and the channels one to four. One of these channels

is visible on-screen, all others are off-screen. You use the

left and right arrow keys to get the desired channel

on-screen. Because all channels are always active, an

off-screen channel may receive date any time.

"OnScreenChannelId" defines the color attributes of the

on-screen channel, "OffscreenChannelId" defines the color

attributes for an off-screen channel which has not got new

received data, and "OffscreenChannelIdData" defines the color

attributes for an off-screen channel which has got new data.

Whenever an off-screen channel receives new, unseen (because

it is off-screen) data, then, to give you an indication, the

color attribute changes from "OffScreenChannelId" to

"OffscreenChannelIdData".

"SentTextInReceiveWindow"

In all modes, the text you type is echoed in the lower

(transmit) window.  In the packet mode, when you send the

text by typing a carriage return, the text is moved to the

upper (receive) window, and the transmit window is cleared.

THS/PHS uses different attributes so you can easily

distinguish the text which you sent from the text you

receive.  By default, the text sent has the same text and

background color as the received text, but is intensified.

This is what you get if you specify "SentTextInReceiveWindow

= Default". If specify a color instead, THS/PHS will use

these color attributes.

Example:  if you have defined "ReceiveWindow_Text = Green on

Black" and you want your text reversed, not intensified, then

define "SendTextInReceiveWindow = Black on Green" here.

"SentTextInTransmitWindow"

In the non-packet modes, the entered text for transmission is

sent to the PK-232 word-by-word and you get the echo in the

receive window from the PK-232 (echo-as-sent) when it is

actually transmitted.

For to get an indication up to which part of your text has

already been sent to the PK-232 (and not yet transmitted),

that part is highlighted in the TRANSMIT window if you

specify "Default". If you prefer another presentation, you

can experiment with color values.

Mandatory, PHS only.

ERROR REPORTS

�������������

Whenwever PHS detects an error, it will open a window and display an

error message and optionally parameters. Note that it is not always

possible to recover from these errors. The message types are

RS-232 Port Overrun error

RS-232 Port Parity error

RS-232 Port Framing error

RS-232 Port BREAK interrupt

Hostmode waiting for SOH: timeout

Hostmode waiting for databyte: timeout

Hostmode excessive input, no ETB

Unknown error during input, code: nnnn

The RS-232 errors are signalled by the OS/2 communications device

driver.

Parity and framing errors, or break interrupt may indicate a chip

problem, this could however be on either end of the serial line. Try

to use another port on the PC, and/or reduce the transmission baud

rate.

Whenever you get an overrun error this indicates that the your PC is

not fast enough to process the incoming characters. This can be

solved by either reducing the frequency in which the incoming

characters appear, i.e. by reducing the communications baudrate, or

by using a more sophisticated serial communications controller chip,

or by increasing the CPU speed.

The "hostmode" errors signal a problem with the PHS-PK-232 hostmode

communication. Up to now I have ever only seen the first one (timeout

waiting for SOH), and only during startup of PHS. A hardware reset of

the PK-232 and or a hardware (!) reset of the PC cured it (on the PC

side, a CTRL-ALT-DEL will not help).

DECODING OF NET/ROM FRAMES

��������������������������

PHS decodes Net/Rom routing broadcast messages and the inter-node

frames into a plain English text format. For an interpretation of the

various variables shown, please refer to "Net/Rom Version 1.3

Documentation" from Software 2000 Inc.

FINAL REMARKS

�������������

I will be interested in your proposals and comments. Please note

however that I will feel no obligation of any kind. I am a

professinal software designer, but THIS is a hobby project.

My address is:          Peter H. Heinrich

HB9CVV

Allmendstr. 25

CH-2562 Port

Switzerland

You can also contact me on CompuServe [71470,32].

REVISION HISTORY

����������������

2.xx    1988            Releases of THS (DOS)

3.00    October 1989    Initial release (PHS for DOS)

4.00    October 1991    Beta release (PHS/THS for OS/2)

February 1992   Inital release (PHS/THS for OS/2)

5.00    March 1993      Internal release

6.00    May 1993        Beta release (PHS/THS for OS/2 2.x).

PK-900

������

******************************************************************

* A note to PK-900 users: It is highly recommended NOT to use    *

* the PK-900 at a communication line speed of 19200 baud, but    *

* use 9600 baud only. The PK-900 seems to have problems at 19200 *

* baud (which can be seen by the use of a line monitor).         *

******************************************************************

To enable the proper operation for a PK-900, you must set a

devicetype of 4 in the configuration file.

The statusline for the PK-900 will be slightly different to reflect

the operation of the second modem ("Radio 2"). This is what you see

when Radio 1 is in Amtor-Listen mode and this channel is selected:

AMTOR LISTEN   � RECEIVE  � Phasing     �              �    � MABCD � 19:09:03

The MABCD string contains the channel indicators of the four

channels of Radio 2 (A, B, C, and D) and the monitor channel for

Radio 2 (M). To select one of these channels, use the left or right

arrow keys. The statusline will change to:

1    � VHF � UA:00 RTY:00 � DISCONNECTED              �    � MABCD � 19:15:21

and one of the indicators (MABCD) is highlit to show which channel

is currently selected. The single "1" is the indicator of the Radio

one channel - in non-packet modes, there is only one channel.

If you have a Radio-2 channel selected, the mode-swich command (by

ALT/M) will allow you to change between Packet-VHF and Packet-HF

only - other modes are available only on Radio-1.

If you are on a Radio-1 channel, and switch the mode to Packet, the

status line will be like this:

M1234 � HF  � UA:00 RTY:00 � DISCONNECTED              �    � MABCD � 19:23:24

Again, one of the indicators either on one of the Radio-1 channels

(M1234) or of the Radio-2 channels (MABCD) is highlit to indicate

the currently active channel. Use the left or right arrow keys to

change to another channel.

PHS maintains two different heard-lists, one for each radio of the

PK-900. Whenever you type ALT/H or ALT/G, the heardlist for the

currently selected radio is shown.

When ALT/I is pressed, then PHS shows twelve entries in the selection

list for the PK-900 - CFROM-2, DFROM-2, MFROM-2 and MTO-2 is added for

Radio-2.

Please also remember that most of the packet-related PK-900 commands

have now two parameters, seperated by a slash ("/") where the first

parameter applies to RADIO 1, and the second parameter after the slash

is for RADIO 2. Consequently, if you press ALT-P on a packet-channel,

you get something like this:

���������������������������������� .... ���������������������������������������

��������������������� PK-900 Parameters �������������������͸

�                                                           �

� ACKPRIOR      Y/Y   FULLDUP       N/N   RESPTIME      0/0 �

� AUDELAY       2/2   HID           N/N   RETRY       10/10 �

� AX25L2V2      Y/Y   LITE          N/N   SLOTTIME    30/30 �

� AXDELAY       0/0   MAXFRAME      4/4   SQUELCH       N/N �

� AXHANG        0/0   MCON          6/6   TRIES           0 �

� BARGRAPH        0   MID           0/0   TXDELAY     30/50 �

� BEACON    E 0/E 0   MODEM         4/4   USERS         2/2 �

� BRIGHT         50   MONITOR       6/6   WHYNOT          N �

� CHECK       25/25   MRPT          Y/Y   XMITOK          Y �

� CMSG          N/N   MSTAMP          N                     �

� CONPERM         N   PASSALL       N/N   <F1>  HELP        �

� DWAIT       16/16   PERSIST     84/84   <ESC> EXIT        �

� FRACK         5/8   PPERSIST      Y/Y   <F10> SET PARAM   �

� FRICK         0/0   RELINK        N/N                     �

�                                                           �

�����������������������������������������������������������;

1    � VHF � UA:00 RTY:00 � DISCONNECTED              �    � MABCD � 19:30:29

Eable acknowledgement priority (Y/N)

���������������������������������� .... ���������������������������������������

In the Config-file, there are now five more keywords - ChannelM2size

and ChannelAsize to ChannelDsize for the new channels of Radio-2.

Please note that the help-text for the commands is no longer

complete.  A long time ago I got the PK-232 command summary from AEA,

this was then used for the helptext. Unfortunately, it is no longer

possible to get this from AEA, and I do not like to type always the

latest command summary from the manuals. I did it for some commands,

but got tired to do this for all. Thanks for your understanding...

When the RESETdeviceonexit parameter of the Config-file is ON (which

is the (recommended) default) then PHS switches both radios OFF on

exit, and (always) ON when started. This means that the RADIO 1 and

RADIO 2 indicators of the PK900 LCD panel go off when PHS exits. In

case you want to operate the PK900 with another program, you must

first issue a "RADIO 1/2" command to enable both radios (modems).

Version 6.01 and higher has the ALT-1 and ALT-2 keys to toggle

Radio-1 or Radio-2 on/off. However, it is possibly not wise to

switch a modem off during a qso...

����������������������������� end of file �������������������������������������

so far.