Handy->8pin adapter

De DG8FZ @ DB0GV

HANDY         ADAPTER         ICOM-8-PIN     EACH ORIGINAL ICOM MIC OR :

2.5mm      +-2--------------------O 2        +5V (only  IC02E + comp.)

+     e       c                    (2.5mm C500 : 2=gnd , 7=n.c. !)

1*2*7=====+-1-----.   /----------O 1

+        > /  TUP                 Mic  (don't block current !)

+     b -----

+         +----10K-*---O 5        PTT

+                  I

+-7-+            100pF

3.5mm          +--------------*---O 6+7      GND

+-6-+

8*666=====+

+-8--------------------O 8        SPK

73 de DG8FZ @ DB0SAO  3.Sept.1988

CI-V -> V24/RS232

CI-V                             ( TTL-Level (C=64 etc.) )            RS-232

3.5mm female                               .                          female

.         *--------------*-6 DSR

(+5V)   .         *-AK-------+   +-8 DCD

+-----------10k-----------KA---*------------*--*----78L05-*-KA--9 +12V

!               +----*---*-----*     .    16!         !   K

!               !    !   !           .    .''!       ---  A

!   +--*-!    14!    !   !           <- -9inv8- - - ------*-<---4 RTS

!   !  S      .'''1--+   !           .    '..!MAX232

!   A  G-10k*3 xor!      !           .    .''!

!   K  D    ! '...2------(-----------<--12inv13-------------<---2 TXD

O    !   !  !    !            !           .    '..!

! ^--*---*--*    +6'''.       !           .   !''.

!       D1  !     ! xor4-*-*--(----------->--11inv14------------->---3 RXD

---          +----*5...'  K !  !           .   !..'

xor = 1/4 4070    !       A 10k!           .   !''.

inv = 1/4 MAX232  !       *-+  +8'''.    +->--10inv7-------------->---5 CTS

FET = BS 170      !       !     !xor10---+ .   !..'

D1  = ZPD 18V     !       *-----9...'      .     !15

D2-5= 1N4148      !      0u1      !7       .     !

*   connection    !   !---*---*---*--------------*--------------------7 GND

(   no connection !           !            .

--! ground        +------+    +-12''.         MAX-232 C's

-!]- =  --!!--  22uF     !      !xor11       16--!]-- 2 (+10V)

+            +------13..'         5--!]-- 4

(-10V) 6--!]--15

3--!]-- 1

300/1200/9600bps ; 1 stop-bit ; no parity

transfer data from 8-bit-ascii -> hex  and read backwards.

start-flag destignation surce command info          endflag

!            !     !       !             !

FE FE          0        20      0    00 00 50 45 1 FD

.  .  .  . 100MHz

.  .  .  1MHz

.  .  10KHz

example for qsy                        .  100Hz

to 145.500 MHz                         1Hz

command: (not all command's are used by all rig's)

No.   Name            Info

00    set freq        see example

01    set mode        00 - 05

02    read freq-range  -

03    read freq        -

04    read mode        -

05    set freq        see example

06    set mode

07    set vfo

07 00 set vfo A

01         B

A0         A=B

B0         A(M) <-> B(s)

C0 dual watch off

C1            on

D0 main access

D1 sub

08    set memory

xx            CH

09    MW

0A    M => VFO

0B    M-clear

0C    read offset

0D    set offset

00 scan stop

01 scan start (p/m)

02 p-scan start               !

03 delta-F-scan start         !

04 auto-w- scan start         !only 781/R9000

12,13,22,24,42,a0,aa,a1,a2,a3 !

0E A4,A5,B0-2,C0,C1,D0-3         !

0F 00 SPLIT OFF

01       ON

10 simplex    !

11 duplex -   !no existing rig

12        +   !(if my manual is right)

10 00 TS off          !

01    on (data 1)  !only R9000

xx    on (data xx) !

11 xx ATT xx-db       !

12 0/1 ANT off/on     !

13 0/1 speech all/freq!

14 01 AF GAIN SET     !

02 RF GAIN SET     !

03 SQL LEVEL SET   !

15 01 READ SQL (open?)!

02 READ SIG LEVEL  !

FB                       OK-command (packet was OK and is exicuted)

FA                       Not Good command (the packed's data is unexcutabel)

FC                       packet was not ok  ( collision )

remoteable family's: (some need an IC-UX-14)

R71,751,751a

271,471,1271

761

735

R7000

275,475,575,1275

781

725

R9000

765

CSMA/CD or how icom made the handshake :

if on the bus is not the transmitted info, this is caused by a colision.

so the last packet is cancelt by  'FC FC FC FC FC',

and resend later.

you can check this by software (like icom does) (4070 pin11-->max11(only) )

or by hardware (like the cirquit above).

73's de karl dg8fz @ db0sao.deu.eu

IC-xx71 FM-VOX

de DG8FZ @ DB0GV

How to make the VOX of an IC-271, 471, 1271 usable for FM:

1st type of 'main-board':

The only thing to be done is to add a diode such as 1N4148 or BAX13 on the

MAIN board on the position D74.

When D74 is inserted ... the VOX works in FM also.

The diode is on the shematic diagram of the MAIN board but is not mounted on

the PCB in the factory.

73's de Stefaan

ON4BY @ ON4HU

16/03/1990  18.31 h

2nd type: (on this board a D74 is present)

Disconnect the kathode of D69 from its old postition.

Its new position is the kathode of D50 or D51

NAME! POSITION ON MAIN BOARD! (OLD) FUNCTION!

D69 ! near 'CW-DELAY'.      ! 'FM -VOX-OFF' ! FM 8V

D50 ! near 'RF-YGR-Unit'    ! 'USB-VOX-ON'  ! USB8V

D51 ! near 'RF-YGR-Unit'    ! 'LSB-VOX-ON'  ! LSB8V

73's de Karl

DG8FZ @ DB0SAO

20/03/1990   16.52 h

IC-xx71 tuning steps

de DG8FZ @ DB0SIF

The IC-271 FM-Step Selectmatrix on the 'LOGIC'-bord is:

IC6 Pin 11 -*------*------*------+

!      !      !      !

A      A      A      A          (1N4148 or equ.)

K D19  K D20  K D21  K 'D21a'

!      !      !      !

IC12 Pin    3      4      7      8

Installed Diodes are maked with X :

D19 D20 D21 D21a   Steps

10  Hz

X                100  Hz

X                   1    KHz

X   X               5    KHz

X          10    KHz

X       X          12.5  KHz

X      25    KHz

X   X    1       MHz

IC-471 and IC-1271 are equivalent. D21a may be named D34.

73's de Karl ( dg8fz @ db0sao.deu.eu )  24.Okt.91

IC-xx71-ram-adapter (1. update)

de DG8FZ @ DB0SIF

How to read/program the ICOM-battery-RAM by EPROMER:

Adapter ICOM-RAM <==> 2764-Programmiersockel

12.dez.89      ....................

.       R    5V  20.------ 28  +5V the factory programs

Q4-7 16-19 -+  .       A  CS    19.---(*)         the lower 255 Nibbels...

0V  14 ----+-- 1 E    M  AD 9  18.------ 24  A 9

.2         AD 8  17.------ 25  A 8 you find the frequency-

+5V  28 --o..o- 3 WP   U  AD 7  16.------  3  A 7 range reading the 'hexdump'

Q3  15 -------.4 D3   N  AD 6  15.------  4  A 6 backwards.

Q2  13 -------.5 D2   I  AD 5  14.------  5  A 5

Q1  12 -------.6 D1   T  AD 4  13.------  6  A 4 mni tnx 2 dl8scu @ db0aaa,

Q0  11 -------.7 D0      AD 3  12.------  7  A 3           df8wu  @ db0lj ,

Prog 27 -------.8 WR'     AD 2  11.------  8  A 2           dc4fe          &

testet with:   .          AD 1  10.------  9  A 1           dl8zaw @ db0sif

IC-751 , 271h, . upside   AD 0   9.------ 10  A 0

471e,1271a. ....................

CE--20-inv.--

OE--22-      nand--(*)    use a 4011 for the nand's and the inverter.

nand--             dont forget to set the unused 4011 inputs high.

PGM-27-

If your eprommer cancels operation with a 'no eprom inside'-error

contact each eprommer-pin to a normal 2764 but

dont connect the pins  11,12,13,15,27 of the normal eprom,

dont connect the pins  16-19 to 0V as shown above,

program the 2764 to all bytes = low before.

73's de Karl  (dg8fz @ db0sao.deu.eu) 12.Dez.89, 21.Okt.91

TITOLO: TUNING STEPS CHANGE FOR ALL MODE ICOM IC-275/475/575/1275

The IC-275/475 FM-Step selectmatrix use these diodes on the front unit:

(type 1SS53)  D41, D42, D43, D44, (D45), D46

Note: diode D45 is not present but solder holes exist for it on the board

just between D44 e D46

installed diodes are marked with X :

(*)     (*)

D41 D42 D43 D44 D45 D46 D48    Steps     DB4 DB5 DB6 DB7

.   .   .   .   .   .   .    10.0  Hz

.   .   .   X   .   .   .   100.0  Hz    1   0   0   0

.   X   .   .   .   .   .     1.0 KHz    0   0   1   0

X   .   X   .     5.0 KHz    1   0   1   0

.   .   .   .   X   .   .    10.0 KHz    0   1   0   0

.   .   .  (*)  X   X   .    12.5 KHz   (*)  1   1   0

.   .   .   .   .   .   X    25.0 KHz    0   0   0   1

X   .   X   .   .   .   .     1.0 MHz    0   1   0   1

(*) To set a step of 12.5 KHz in FM mode (TS button off) it is not

necessary disconnect the diode D44, just insert the diode D45

(IN4148 or similar) near D44 and "voil les jeux sont faits".

Unscrew the two lateral screws and unlock the front panel.

Remove also all the connectors of the front board and after that

unscrew ALL the screws of the metallic shield covering the front board.

It seems too much complicated insert the diodes from the component

side of the front board, so it is better insert D45 from the solder side.

'73 de IK1OOM

IK1OOM @ I1YLM (Roberto ORLANDINI - Torino ITALY)

*****************************************************************************